Before the

Federal Communications Commission

Washington, D.C.

In the matter of:

| Revitalization of the AM Radio Service | ) ) MB Docket No. 13-249 ) FCC 18-139 |  |
| --- | --- | --- |
| Second Further Notice of Proposed ) Rulemaking ) |  |  |

## COMMENTS OF KINTRONIC LABORATORIES, INC.

(Prepared by: Stephen F. Smith, Ph.D., EE)

The domestic and international broadcast radio engineering, design, and manufacturing firm of Kintronic Laboratories, Inc. (“KTL”) hereby submits these Comments in response to the Commission’s recent action on AM Revitalization, Second Further Notice of Proposed Rule-making (SFNPRM), dated October 5, 2018, in the above-captioned proceeding.  In that Notice, the Commission has solicited comments on its various further specific proposals to revitalize AM radio and also invited submission of further proposals. Based on experience from the  65-year history of our firm and its founder, Louis A. King, MSEE, PE,  providing engineering consulting and product services to the licensees of U.S. AM radio stations as well as many international broadcasters, we intend with these comments to provide focused analyses of the Commission’s specific proposals related to AM transmission standards and also add to the discussion with further proposals we believe to be essential for AM revitalization. Our comments will focus on several over-arching needs for AM radio, plus specific Commission policy changes that will facilitate improving the coverage of all stations in the existing AM band.  We believe that the Commission's stated goal of truly revitalizing the AM broadcasting service can only be achieved by a concerted, multi-faceted approach to this complex technical, economic, and policy challenge. Ultimately, the American listening public will be the real beneficiary of these changes.

Executive Summary

In this section we summarize our final conclusions and recommendations to the Commission as Comments to this action on AM Revitalization (FCC 18-139).

(1) The proposed radical changes to the AM protection contours are totally unjustified. For nearly a century, the existing Rules have adequately protected AM stations from mutual co-channel and adjacent-channel interference and have fostered a robust expansion of the AM broadcast band, providing a key service to the American public. The traditional engineering standards originally promulgated by the Commission for the AM band, based on realistic physics and engineering considerations, have been similarly adopted worldwide by the ITU and WARC organizations and are fully endorsed by the world's radio engineering bodies. The existing U.S. contour  protection ratios (26 dB co-channel and 6 dB adjacent-channel) have been adopted verbatim in the New World (WARC Region 2), and with a few very minor differences, have been embraced in Europe and elsewhere (30 dB daytime and 27 dB nighttime co-channel for WARC Regions 1 and 3). [The 40-dB figure adopted by the EBU is referenced to a 1.0 mV/m contour, which translates to 34 dB D/U (desired-to-undesired signal ratio) at the existing standard 0.5 mV/m contour (current §73.37 rule for Classes B, C, and D stations)]. We herein cite the specific ITU Recommendation document ITU-R BS.560-4 (1997):

In § 1 of this Recommendation, a value of 40 dB is given for use in bands 5 (LF) and 6 (MF) for co-channel transmissions.

With this value of RF protection ratio, high quality reception is possible. For planning purposes however, it may be necessary to adopt lower values. This problem has been studied by the EBU and in Japan. The values that have been proposed are 30 and 26 dB, respectively, and in fact, a value of 30 dB was agreed by the Regional Administrative LF/MF Broadcasting Conference (Regions  1 and 3) (Geneva, 1975), whereas 26 dB was used by the Regional Administrative MF Broadcasting Conference (Region 2) (Rio de Janeiro, 1981).

The current U.S. rules are thus strikingly consistent with the corresponding ITU/WARC recommendations. Unfortunately, the major alterations to the protected contours suggested by the Commission staff and several Commenters in this proceeding are in fact based on two highly faulty assumptions. These are: (1) that the increasingly degraded signal-to-noise ratio (SNR) in the AM band cannot ever be remedied; and (2) that all AM receivers will continue to have very narrow bandwidths (~ 2-3 kHz). Neither condition is in reality true: (1) the "SNR" issues plaguing AM radio can be rapidly resolved (in probably 2-5 years) by aggressive Commission enforcement of the existing Part-15 and -18 Rules for Intentional, Incidental, and Unintentional Radiators (as detailed in the following "Noise" section of this document), coupled with the limited lifetimes of cheap, noisy, non-compliant switching power supplies and other consumer devices; and (2) the  increasing presence in the consumer market of wideband-capable AM receivers, including DSP-based analog receiver units with selectable or adaptive bandwidths of up to 9 kHz (e.g., the Tecsun PL-880 portable), as well as a variety of digital (HD Radio®) products. Wideband AM formats, including both CQUAM analog and the new Xperi/Ibiquity all-digital on-channel format, would be severely impacted by the proposed changes to the existing contour-protection rules, as summarized below. Only by willfully ignoring the need for wider-bandwidth radios (both analog and digital) can the proposed radically increased levels of co-channel and adjacent-channel interference be justified. In addition, the proposed degradations in 2nd- and 3rd-adjacent interference protections will grossly overload many existing receivers, including HD digital units, which have been designed to the post-1991 allocation scheme [1,2].

Existing versus Proposed Section 73.37 Protections:

| Frequency Separation (kHz) | Existing Contour (Class A) (mV/m) | Proposed Contour (Class A) | Existing Contour (Classes B, C, D) (mV/m) | Proposed Contour (Classes B, C, D) (mV/m) | Contour of any other station (mV/m) | Contour of any other station (mV/m) | Potential increase in interference compared to current §73.37 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.1 | 0.5 | 0.025 | 0.025 | 0.500 (Class A) | 0.500 (Class A) | +28 dB |
|  |  |  | 0.5 | 0.5 | 0.025 (Class A) | 0.025 (Class A) | +28 dB |
|  |  |  | 0.100 | 0.100 | 2.0 (Other classes) | 2.0 (Other classes) | +24 dB |
|  |  |  | 2.0 | 2.0 | 0.100 (Other classes) | 0.100 (Other classes) | +24 dB |
| 10 | 0.1 | 0.5 | 0.5 | 0.5 | 0.5 (Class A) | 0.5 (Class A) | +6 dB |
|  |  |  | 2.0 | 2.0 | 2.0 (Other classes) | 2.0 (Other classes) | +30 dB |
| 20 | 5.0 | 25.0 | 5.0 | 25.0 | 5.0 (All classes) | 25.0 (All classes) | +28 dB |
| 30 | 25.0 | --- | 25.0 | Unlimited Interference | 25.0 | Unlimited Interference | +Infinite! |

Perhaps the most unfortunate 	aspect of the proposed (degraded) protection limits is simply the confusion between noise (natural and/or manmade) and interference (from other radio stations). The stated impetus for permitting these higher transmitted power levels is actually to improve the SINR [signal-to-(interference+noise) ratio, S/(I+N)], where the denominator represents the sum of the RF interference power and the external ambient noise power, which is itself totally independent of the broadcast signals' magnitude! Unfortunately, the thinking behind the revised protection contours utterly fails to distinguish between the actual noise (from atmospheric plus extraneous manmade sources) and the resulting severely increased station-to-station interference. The AM band needs better SNRs, not worse SIRs (signal-to-interference ratios)! Obviously, nothing can be done to regulate natural noise, but the Commission can and must enforce its own existing Part-15 and -18 regulations to bring harmful emissions of electronic devices in the AM (and other radio bands) into meaningful compliance. In brief, as to the contours, "if it works, don't fix it"; relative to the noise, regulate it down to a de minimis level via administrative action. Indeed, no Rules changes (§73) are necessary, just active enforcement of those already on the books (§15 and §18).

Overview and Detailed Discussion. We have reviewed the Second Further Notice (and its parent 13-249 Notice of Proposed Rulemaking [NPRM] of October 31, 2013 and the subsequent First Report and Order, Notice of Proposed Rulemaking [FNPRM], and Notice of Inquiry, dated October 23, 2015), and we wholeheartedly support the Commission’s goal of revitalizing the AM radio service. AM radio constitutes the most bandwidth-efficient broadcast medium and provides an essential service to many Americans, particularly in rural and remote areas, and those traveling in the vast expanses of this nation. We strongly concur with the full Commission's efforts in this regard, especially Chairman Pai's efforts to champion this thrust, and with past Commissioner Clyburn's recognition that AM provides a unique venue to facilitate female and minority media management and ownership, as well as to provide vital programming diversity for the American public, particularly in niche markets and demographics. AM radio, due to its generally lower capital requirements, can also provide a realistic setting for family-based, community-focused  station programming and ownership, especially in smaller localities. AM radio is truly a national resource, a source of unique voices, and one that we can ill afford to abandon, particularly in light of its unique propagation characteristics and tremendous reach, especially in times of local, regional, and even national emergencies. Truly, this AM Revitalization action has the rare  potential of conserving a unique national resource.

Since the original NPRM over five years ago and the FNRPM over two years ago, we have as yet seen no substantive response from the Commission on the two salient issues we (and  others) have identified as key to the survival and ultimate viability of the AM band   (1) the noisy electromagnetic environment; and (2) the concurrent failure of the consumer-products industry to provide the listening public with high-quality AM receiver systems (comparable to their FM counterparts), particularly in the areas of sensitivity, selectivity, noise rejection, and audio bandwidth. As we cited, these two effects are in fact closely interrelated, since the steadily increasing noise floor in the AM band has materially contributed to the unfortunate trend to reduce AM receiver bandwidths even further than those typical in the 1960s and '70s. It has been all too easy for the receiver manufacturers to simply reduce overall receiver bandwidths down to even 2-3 kHz (sometimes worse than telephone grade!) to address the pervasive issues of electromagnetic interference (EMI) noise from power lines, LED- and fluorescent-lamp ballasts, personal computers, consumer devices, battery chargers, and the like, not to mention broadband static impulses from lightning and increased adjacent-channel and alternate-channel interference from more recently allocated AM stations. Added on top of all this is the progressive trend in the automobile industry to replace metal body parts with plastic (which worsens EMI shielding), adapt windshield-type antennas (which provide markedly poorer reception performance for both AM and even FM), and add a multitude of noise-generating microcomputers for engine control, antiskid braking systems, and the like. An additional upcoming challenge lies in new vehicle electric-drive systems, which can generate even more noise in the AM broadcast band. An additional upcoming challenge lies in new vehicle electric-drive systems, which can generate even more noise in the AM broadcast band. Some automobile manufacturers of all-electric or hybrid electric vehicles have properly addressed noise suppression in AM receivers, while others have used the increased noise environment as an excuse to remove AM receivers from their vehicles.  Low cost noise suppression technology is available to eliminate the AM band noise emission produced in any electric drive vehicle.  The net result for the public has been AM radios with very low audio and reception quality that have resulted in a steady decline in AM radio listenership.

It is therefore imperative to the sustainability of AM radio that the Commission strongly encourage (or, via legislation, even mandate) significant improvement in consumer AM receiver systems. Without the implementation of advanced, DSP-based consumer-receiver features to address the severe noise, interference, and bandwidth challenges to good, clean AM-band reception, the appeal of AM to the public will inevitably be lost.

In the time since the NPRM issuance, we have been informally apprised by staff that though the Commission would be willing to assist in certification of receivers for minimum performance standards, the agency could not consider mandating AM receiver performance without specific signed legislation passed through Congress. As a result, our company has engaged in two separate internal development projects to design advanced, high-performance DSP-based AM/FM receivers, one a professional instrument-grade unit, and the second (collaboratively with a partner firm) to produce a high-volume, low-cost  software-based DSP high-performance consumer-type unit, initially in table-top format. The latter unit was also successfully demonstrated at NAB 2017. Both designs will offer AM receiver parity with the dominant FM band to enable the user to make listening choices on a much more level playing field. The relevant technical receiver standards to achieve effective parity include: (1) low internal noise floor; (2) high overall RF sensitivity, selectivity, and dynamic range; (3) highly effective noise (EMI) rejection; (4) full 10-kHz audio bandwidth capability with low distortion; and (5) stereo capability [both AM and FM].

Since the initial NPRM, we have also demonstrated wideband stereo AM reception (using music) versus standard FM stereo at our NAB booth in both 2014 and 2015, at the Association of FCC Consulting Engineers (AFCCE) meeting in Washington in the fall of 2014, and two separate  demonstrations (also in the fall of 2014) at the Commission facilities in Washington to Commissioners Pai and Clyburn and multiple staff members from the Commissioners' Offices, the Media Bureau, the FCC lab, and the Enforcement and International Bureaus. All who auditioned the demos agreed that wideband 10-kHz AM stereo was (surprisingly) very competitive in discernable sound quality with FM (some even preferred the AM!) and would serve to boost AM listening when made available to the general public. This need for adaptive, wideband AM reception to better compete with FM underscores our strong belief that the overly pessimistic perspective held by many in the engineering community that wideband AM receivers will never again be deployed will likely  lead to egregious policy errors in formulating new AM protection standards. For example, in addition to the very large changes (12-14 dB) proposed in the protected contours for Class A, B, C, and D stations, the proposers' usual response is to ignore or reduce the role of adjacent-channel contributions to the overall interference picture. Unfortunately, for even narrowband receivers, this interference is much more than just-audible 10-kHz carrier whistles; the full-bandwidth (10-kHz) nature of the overlapping, inverted-spectrum sideband interference from adjacent channel stations, virtually all of which are very heavily modulated and pre-emphasized, results in an annoying "buzzy", syllabic-rate high-frequency splatter that directly degrades the audio from the selected station. If there is significant signal from both adjacent channels (e.g., as frequently encountered at night), the splatter is doubled, with the two interfering inverted-spectrum audios superimposed. (Second-adjacent stations are much less problematic, as their sidebands are significantly attenuated by the I-F selectivity of virtually all current receivers). In our local area, there is a very long-standing 1-kW Class-C station on 1400 kHz, and two nearby (~31-40 miles) adjacent-channel stations, one with 2.5 kW on 1390, and another with 500 W on 1410. At the reference location, 22 miles out from the 1400 station, a strong, low-noise daytime signal is readily received from the main station, but significant annoying crosstalk from the two (lower- and upper-) adjacent-channel stations is constantly received on a typical narrowband but sensitive OEM Toyota radio. Since these stations were allocated prior to the adjacent-channel Rules changes in 1991, their 0.5-mV/m contours are supposedly no more than 0 dB relative to those of the 1400 station, but the adjacent-channel interference (ACI) clearly degrades the 1400 signal much more than does the average power-line noise. On a wideband AM tuner at the same location, the ACI renders the 1400 station almost unlistenable with a 10-kHz I-F bandwidth, and marginally listenable with a 5-kHz bandwidth, both with substantial, constant modulation splatter. Except with advanced DSP techniques such as selective-sideband, this form of ACI is exceedingly difficult to filter out. Circumstances like this remind us that ACI, due to its sideband spectral overlap with the desired station,  is a tremendous detriment to good-quality wideband or even medium-bandwidth (e.g., 5-kHz) reception; if we are to have any hope of decent AM audio quality, the ACI must still be controlled (at worst) to the post-1991 levels. With the proposed changes to the AM protection contours in the SFNPRM action, and the anticipated mutual power increases, the ACI would typically increase by 6 dB (at the traditional 0.5-mV contour) and could be as high as 12 dB (1.0-mV protected contour), or 18 dB (2.0-mV protected contour). We hold that this is unacceptable for AM to ever be a quality reception medium.  We do not disagree with the logic of our engineering colleagues in suggesting higher levels of permissible ACI (if we assume only narrowband voice programming into narrowband receivers), but we still firmly believe that to succeed, a large proportion of  AM stations must also be able to successfully broadcast reasonably wideband (6-10 kHz) music programming. We certainly accede that wideband AM radios (e.g., the AMAX units, which have been somewhat available in the past) are currently difficult to obtain, but we believe the new DSP-based consumer unit we are collaborating on (along with other units being developed and now available on the market) will within a few short years change all this.

As we cited in Further Proposal 6 in our original NPRM Reply Comments of March 2014, numerous Commenters, especially those in the engineering community, have proposed several substantive changes to the Commission's Rules for co-channel protection of standard contours on virtually all classes of stations, based on the overriding assumptions that neither the levels of RFI nor average AM receiver performance will ever improve. Given these pessimistic assumptions, the arguments presented are logical, but we fundamentally and emphatically disagree with that thesis. For the foreseeable future, we strongly encourage the Commission to defer any such irreversible allocation actions until all four of the initial measures we have proposed (q.v.) have truly had sufficient time to work. If these protection limits are reduced now, there will be no later chance of ever recouping the lost coverage areas — the zones previously denied by noise will simply now be squashed by added co-channel (and adjacent-channel) interference from other stations.

NOISE    THE "ELEPHANT IN THE ROOM"

Historically, for many decades the foundation for the success of AM broadcasting has been due to the  Commission's establishment of rational allocation standards that have supported a workable signal-to-(interference+noise) ratio (SINR)  in the vast majority of listening environments. In fact, the "noise"  (random + impulse noise from lightning and static + broadband noise from man-made sources) and "interference" (from other radio stations) are all truly independent of each other and must be dealt with individually. In the past (perhaps 30-40 years ago), noise contributions from power lines and the like were generally quite modest, as Utilities were diligent to maintain their systems in very good working order; bad AC power junctions and insulators were detected and repaired promptly. Even clear-channel AM signals, from many hundreds of miles distant, were readily receivable on both portable and home radios, both indoors and outside, without noticeable local utility interference; in fact, the biggest reception problem then was usually skywave fading! Now, the noise situation is drastically worse, as we addressed earlier in this Proceeding. Though certainly not reflective of the current Members of the Commission, the agency has for many decades been apparently distracted by  a multitude of non-AM broadcast matters and has essentially completely ignored the growth of EMI/RFI issues, which have most severely impacted the AM band, but have also caused significant harm to the amateur radio bands and have begun to impact both low-band VHF-TV and FM broadcasts (as per SBE and NPR sources). Establishment of the TAC committee is a good step forward, but a concerted move to solve the noise-pollution issue is nearly a generation overdue!

The noise term (N) in the SINR expression above, which used to be much less than the interference term (I), is now often dominant, especially in dense urban environments. This shift has now led many broadcast engineers to think only in terms of SNR, essentially ignoring the interference term. Unfortunately, both terms are still valid and must be considered. Many decades ago, the Commission set up comprehensive, honest, yet conservative AM broadcast interference standards that have since been adopted worldwide, most notably by the ITU and WARC agreements. Clearly, man-made noise was lower back then, but we believe that abandoning the time-tested allocation Rules (intended to minimize mutual interference and so preserve signal coverage) and trying to solve the SINR dilemma by arbitrarily raising transmitted power levels is fraught with severe implementation consequences, many of which are, of course, unintended. These will be discussed further in the Comments section of this document.

We now reiterate a portion of our Reply Comments in response to the original 13-249 NPRM:

Further Proposal 1  Immediately Enforce Part 15 Rules on Unintentional and Incidental Radiators for Electric Utilities, Telephone, and Cable Companies and for Consumer Devices that Degrade AM Radio Reception

Very obviously, the gradual growth of EMI from overhead electric power lines (at all voltage levels), telephone lines and cable TV lines, and a variety of consumer devices has been a tremendous detriment to AM broadcast reception, as we have cited in our Reply comments to the NPRM, as well as several others in this Proceeding.  Part 15 of the Rules clearly sets reasonable limits for both Conducted and Radiated emissions, both within the AM band and elsewhere. Surprisingly, the Commission has never adopted parallel Rules for Incidental Radiators such as power lines; the obvious solution is to draft and enact new Part-15 regulations to put Incidental Radiators in the same category as other noise sources. Although AM-band emissions are especially problematic, wideband radiation from these sources can also affect AM receivers, amateur radio operations, and other communications users; such illegal emissions are rightfully deemed "harmful interference" and have been universally understood as such in the communications field. The proliferation in Utilities and general industry of bad high-voltage line insulators, transformer bushings, transient protectors, and line/ground connections, not to mention BPL transmissions, has led to broadly distributed degradations in AM radio reception, particularly since in most cases power lines follow roads. Although electric utilities are the most common offenders in this regard, telephone and cable TV firms also have caused problems, usually due to DSL and other forms of signal leakage. Most current AM radios are quite susceptible to the impulse-type noise so created; once this  raucous "buzz" even temporarily overwhelms the radio, the listener is strongly prompted to switch to FM or another programming source.

A brief description of the nature and physical distribution of these "hot-spot" noise sources is provided below. The temporal nature of most power-line related noises, such as arcs, breakdown in insulators, and such are usually spikes or impulses which occur at rates equal to multiples of the basic AC power-line frequency, most often 120 Hz, since arcs generally occur each half-cycle of the line voltage. The comparative spectral plots in Appendix A show typical switching spectra for older magnetic systems such as fluorescent ballasts, switching at 60/120 Hz (in blue) versus the much higher rates of modern electronic switching power supplies (in red). Of course, the power-line noises much more closely resemble the magnetic devices. The noise spectra generally, but not always, fall with frequency as 1/f.  Nevertheless, both types of systems can significantly pollute AM radio signals, though the emissions from electronic switchers can easily reach even UHF frequencies, especially if the units have poor or no shielding and filtering. Sadly, these signals are not only radiated but often conducted along low-voltage power lines and later re-radiated, often typical in dwellings.

The sources of power-line and other Utility-based noise can generally be considered as relatively small, localized wideband noise generators. These can be modeled as approximately spherical in nature, so that their E (electric) fields fall off roughly with distance (r) as 1/r2, unlike AM antennas, which are planar and have essentially a 1/r horizontal field reduction. The B (magnetic) field goes as 1/r3, so the source is effectively a noise "hot-spot" with very high local amplitude, but with generally short range, and is usually located well above street level (and that of typical vehicle antennas), so one doesn't stay under the spot very long at speed. Nonetheless, if one is stopped at a traffic light, it's a much different story! The main point here is that the amplitude of the "hot-spot" is so high that no sane amount of AM transmission power will quench the noise in the receiver, so all more power can accomplish is to reduce the apparent "size" of the spot source. Of course, each noise "hot-spot" can also couple to the adjacent power lines and so can radiate fairly efficiently along the wires, which all-too-often parallel the roadway. These power-line "hot-spots" are so large in number (yet usually small in total effective area, except when they are coupled into nearly lines) over the typical coverage area of a given station, we have used the term "noise archipelago" to represent the general physical distribution; in the notional drawing in Appendix B, these spots are distributed all around the coverage area, though obviously concentrated near the population center. As the transmitter power increases, the spot sizes decrease in size, but due to the high intensity of the local noise fields therein, the spots are rarely suppressed to effective inaudibility (i.e., below 26 dBc).

In terms of noise impacts on AM reception, it should be noted that modern receiver technology has already demonstrated some 20 years ago very effective suppression of many of these power-line related noise sources by the incorporation of advanced, highly effective RF noise limiting/audio blanking functions [long known to military and amateur radio operators] were initially developed by Motorola for the AM consumer radio market within later-model CQUAM stereo AMAX-compatible chipsets in 1996 (specifically the MC13027/MC13122 combo).  In fact, this blanking was reported to be so effective that when a car radio was driven under an offending power-pole insulator, the noise was hardly noticeable until the blanking duty cycle became so large that the station audio began to drop out entirely! These chips were actually produced for the consumer market, but due to their high cost and the overall decline in AM popularity in that era did not get major use. It should also be noted that these Motorola CQUAM chips supported adaptive-bandwidth 10-kHz notch filters to deal with ACI, driven from special analog on-chip detectors. Thus, these high-performance receiver features are hardly new, but in new receivers with low-cost modern chips and software-defined signal processing can be offered now with a far lower off-chip parts count and considerably less overall cost to the consumer.

Suggested Actions to Combat Noise Emission. The Commission must protect the public interest, along with the licensed broadcasters, by aggressively enforcing its own Regulations. Our view is that a timely Notice of Violation letter to most Utility General Managers and the like should bring a quick response; hefty fines for repeat offenders would assure more than token efforts to resolve these emission issues (more on this later). The specific recommended Rules changes to accomplish this initially are as follows:

- Power distribution lines and associated equipment, traffic-control signals, switching-type power supplies, controllers, and amplifiers should be added to the list of Incidental Radiators [§15.3(n)] and additional engineering guidance for the design of Incidental Radiators added [§15.13].

- Conducted and Radiated emissions limits for Incidental Radiators should be the same as for Unintentional Radiators [§15.107(a)] and for Intentional Radiators [§15.209(a)] from 9 kHz to above 2 GHz. (Overall, a noise source is a noise source!)

- An official, user-friendly FCC website should be set up to facilitate reporting of specific interference incidents, for stations, engineers, and even members of the general public. These reports should be screened by OET and relayed to stations or Utilities as appropriate.

Closer to home, many existing radios and consumer devices (e.g., CD players), not to mention computers, MP3 players, and such, emit very high levels of local RFI  (produced by clock circuits, RF synthesizers, microprocessors, and poor unit design) and thus impair or even preclude proximal AM radio reception. Common problems with Commission (FCC Laboratory) Type-Accepted, Verified, or Certified devices for consumers should be resolved, with some extra effort, through existing channels. Numerous internationally marketed products (from radios to fluorescent ballasts and LED drivers) with RF power-line filters for EU countries, when sold in the U.S.  have filter components missing, in clear violation of Part-15 Rules. Given a few years, this major problem should eventually be soluble through concerted Commission action, particularly on resellers of these devices. It should be noted that the vast majority of these low-cost, low-quality devices will either fail or be obsoleted within 2-5 years and will thus fall out of use. Noisy industrial-grade products improperly sold to consumers, as cited by SBE, are also a growing problem, and must be addressed by the Commission now..

As this noise-reduction effort most directly benefits AM broadcasters, it is incumbent on them to help the Commission enforce the Part 15 regulations. We heartily agree with the 15-142 FNPRM Comments of Kevin C. Kidd, Mark D. Humphrey, and others concerning EMI/RFI sources, and we enthusiastically support Mr. Humphrey's  proposal that the Commission establish a web-based clearinghouse for Part 15-related (and also Part 73) interference complaints. We further suggest that such complaints would be open to stations and concerned individuals, who would be required to submit a detailed "e-report" of the suspected offenses. Such reports could be automatically forwarded to the Enforcement Branch and to relevant Utilities in the area (or offending stations). Appropriate website design should assure that reports are submitted by technically competent persons; in contrast, members of the general, non-technical public would be directed to complain to the local station suffering the interference for problem resolution.

We commend SBE for their Comments in the earlier FNPRM Proceeding, which provide a clear, thorough, and accurate assessment of the many aspects of the current noise issues for the AM band. We heartily agree with all their conclusions, and deem this document as a must-read for all truly interested in the revitalization of AM radio. Their conclusion is simply that  the Commission must immediately roll up its sleeves, take up the gauntlet, perform its legally-ordained regulation and enforcement duties, and not just address, but solve the AM noise issue via enforcement of its own Rules. The broadcasters who pay licensing fees and the listening public await! Though the task to control AM-band (and other) noise at first glance seems nearly insurmountable, we believe that a prioritized approach, coupled with dedicated, consistent, and persistent Commission action, led from the top, will prevail. To achieve this, we suggest the following steps, in sequence:

- (1) Rewrite and adopt new Part-15 Incidental Radiator definitions and emission standards, paralleling those existing for Intentional and Unintentional Radiators, and shifting primary legal responsibility for non-complying Part-15/18 devices to the final seller rather than the end-user;

- (2) Through OET and the FCC Lab, establish approved RFI/EMI measurement techniques and procedures to assist Utilities (power, phone, cable, etc.), consultants, and broadcasters in establishing compliance and interference troubleshooting. Also certify multiple private RF compliance labs to handle most of the routine testing load;

- (3) Provide a web-based clearinghouse to document complaints and resolution actions;

- (4) Issue a friendly enforcement reminder letter concerning the revised Part-15 Regulations to all affected Utilities, phone, and data carriers, based on State Public Service Commission and similar databases;

- (5) After about 90 days, start the process of formally identifying problem areas, beginning with the most egregious cases, as reported by AM broadcasters, the consulting-engineering community, SBE members, amateur operators, and other technical personnel;

- (6) Begin issuance of Letters of Enforcement to offending Utilities and other entities;

- (7) Simultaneously, issue a friendly enforcement reminder letter to all electronics wholesalers and retailers who handle consumer and industrial Part-15 and Part-18 devices (e.g., Walmart, Best Buy, Home Depot, Lowe's, Target, Sears, Amazon, Newark, Digi-Key, Allied);

- (8) After about 90 days, request Part-15/Part-18 Compliance Letters and proof thereof via certified lab measurements;

- (9) As follow-up, have OET and FCC Lab personnel conduct spot tests to sample Part-15 and Part-18 devices sold in stores or on the web;

- (10) Set up a steep monetary fine structure to help finance Commission enforcement costs;  eventually, enforcement could be nearly fiscally self-sustaining.

We opine that with vigorous action, including Part-15/18 enforcement actions and fines, by the Commission, the vast bulk of the Utility-related noise issues on the AM band could be resolved within about 2 years, though there will always be ongoing incidents until the Utilities are sufficiently "trained" to become more-or-less self-policing, with help from the AM broadcasters to resolve local noise problems as quickly as possible. The mess with Part-15 and Part-18 devices will take a bit longer to resolve, as these devices fail and/or cycle into newer devices (which should by then be fully compliant). We predict that these inexpensive, low-quality consumer devices are unlikely to survive more than 3-5 years at most; the industrial-grade units (e.g., LED traffic signals) may be required to have forced EMI/RFI filtering upgrades, which should not be more than 10% of the unit's cost, as a warranty repair.

Comments to FCC 18-139.  In the following Comments on the Second Further Notice of Proposed Rule Making, we emphasize our general agreement with many in the broadcast engineering community, and in particular those of Brian J. Henry (Henry Engineering), who emphasize the concerns of most station owners and engineers in drastically altering the traditional (and worldwide) station protection limits embodied in the AM-band allocation structure. We also offer several specific comments which differ somewhat from those earlier perspectives but which we believe provide significant additional considerations for implementing the necessary actions by the Commission to truly achieve a comprehensive revitalization of the AM broadcast band, to the great benefit of the broadcast industry and the general public.

The major technical issue in the SFNPRM is, in reality, dealing with the high levels of noise in the AM band. The current premise is that the only way to effectively address the deteriorating signal-to-noise+interference (SINR) levels for most AM listeners (at least in more urban/suburban areas) is to boost licensed radiated power levels, at the admitted cost of substantially more station-to-station interference, both co-channel and adjacent-channel. This, as is widely acknowledged, is the result of steadily increasing EMI/RFI from unlicensed and non-compliant devices, along with the effects of non-regulation of power-line and related noises from Utilities and wireline carriers. As we stated earlier, such noise has gone from essentially a background level to a foreground phenomenon over the last 30-40 years, and is currently accelerating in both level and geographic distribution. Clearly, man-made noise was lower back then, but we strongly believe that abandoning the time-tested allocation Rules (intended to minimize mutual interference and so preserve signal coverage) and trying to solve the SINR dilemma by arbitrarily raising transmitted power levels is fraught with severe implementation consequences, many of which are, of course, unintended. The two principal rationales behind these proposed "solutions" offered by the Commission and many in the industry, though logical in an engineering sense, are based on two incorrect premises: (1) that better, more wideband AM receivers will never be available to the public, and thus AM is limited to voice-grade programming only; and (2) there will never be any real forthcoming regulatory relief for all this noise, so the only way to counter the noise pollution is to raise transmit power levels. We understand this pessimism, but we firmly believe the Commission and the industry can and must do better for AM to survive and prosper. Fundamentally, reducing noise via better regulation and enforcement is still the real (and only) solution, not major power increases for the minority of stations who can afford them.

The implementation consequences of the "power approach", as we just stated, of which many are unintended, will now be discussed further. As we see it, among these drawbacks are:

- Increased co-channel and adjacent-channel interference in all cases, with often-significant reductions in stations' interference-free coverage areas;

- Inability, either through power and/or allocation limitations, to maximize facilities to meet proposed new contour protection levels (i.e., 50-kW ceilings);

- Unfair or uneven opportunities to upgrade along with neighboring co-channel stations;

- Inability for many stations to upgrade due to financial difficulties, including capital costs of new transmitter, matching, and antenna systems and increased operating power costs, with little resulting financial ROI;

- Land restrictions which preclude directional arrays to support higher powers;

- The Commission's need to make mutual upgrades "fair and balanced", to avoid increasing the gaps between "have" and "have-not" stations;

- Issues with Class-C stations, who cannot upgrade power due to existing overlapped allocations but who will suffer significantly worse adjacent-channel interference (ACI), both day and night. Again, we remind the Commission that this ACI, due to the total modulation-spectrum overlap with the desired station's sidebands, is very detrimental to the station's coverage, as the typical listener's reaction to this type of interference is a quick tune-out.

As a result, the benefits accrued by some stations able to upgrade their powers must be carefully balanced against the degradations in coverage (and finances) suffered by those who cannot upgrade.

With these perspectives, we now provide our views of the specifics of the proposed SFNPRM technical Rules changes for contour protection of Class A, B, C, and D stations. We fully agree with NAB and others that the Commission must move very carefully and deliberately,  thoroughly understanding all pertinent Commenters' and stakeholders' viewpoints before adopting such profound changes. If indeed the Commission proceeds with these game-changing modifications to their traditional AM protection Rules, the process must be very carefully managed to assure fairness and order to all stations and avoid the unintended consequences we have listed above.

Commission Proposal A – Change Nighttime and Critical Hours Protection to Class A AM Stations

We agree that the 73 large, longstanding Class A stations are worthy of preserving and protecting due to their wide coverage and key roles in major emergencies as disseminators of vital information to the public, especially in scenarios involving widespread electric power failures; in such events, a portable battery-powered radio or vehicle radio may be the only source of news and status information for the affected population. We agree with other Commenters that in special emergency situations, the Commission must fully define an operating protocol to preserve wide-scale broadcasting by the dominant station(s) in these emergencies, coordinated with FEMA and the 77 stations currently designated as Primary Entry Point (PEP) facilities.

Meanwhile, the need for more local AM facilities using the clear channels and those adjacent to clear channels, particularly for new nighttime service to local communities, cannot be ignored. The Commission, in the FNPRM, has tentatively proposed: "(1) all Class A stations should be protected, both day and night, to their 0.1 mV/m groundwave contour, from co-channel stations; (2) all Class A stations should continue to be protected to the 0.5 mV/m groundwave contour, both day and night, from first adjacent channel stations; and (3) the critical hours protection of Class A stations should be eliminated completely".

Our response to these three proposals are as follows: (1) the Commission's proposed Rule to protect the 0.1-mV groundwave contour for co-channel interference (CCI) and to the 0.5-mV contour for adjacent-channel interference (ACI) should probably be retained. It is still possible on the Interstate highways and other areas well away from power lines (except in highly urbanized areas) to receive reasonably clean groundwave signals from Class A stations over 150 miles distant; in our area on I-75 and I-40 we can receive very listenable daytime signals from WSM (Nashville, 650 kHz); WLW (Cincinnati, 700 kHz); WSB (Atlanta, 750 kHz); and WHAS (Louisville, 840 kHz); in some cases, nearby adjacent-channel daytimers limit the reception more than the background noise. Assuming that the traditional co-channel D/U ratio of 26 dB is maintained, we believe that to avoid materially degrading the daytime coverage of Class A stations, which have traditionally maintained large total audiences over wide areas, but to also permit more local service, the protected daytime contour should be continued at 0.1 mV/m. The Commission's traditional  0.5-mV/m protection on daytime Class A from adjacent channels (0 dB) is not ideal but nevertheless still adequate. The Commission must carefully weigh the negative impacts of changing this limit.

We appreciate the Commission's desire to try to remedy the uneven situation that exists between other station classes and Class A stations.  Especially with Class D stations, we recall that the Commission for the sake of more local service has appreciably accommodated them over the years.  In the 1980s, the Commission authorized Post-Sunset Service Authorization (PSSA) for a substantial number of stations, and then recently gave Class D stations distinct preferences in the proceedings to obtain cross-band FM translators. In fact, Class D licensees fully understood the limitations when they acquired their daytime-only facilities. The AM band has always been a multi-tiered service (as FM is), and it is not just for Class A licensees to arbitrarily bear the burden of losing much of their interference protection without some form of compensation for what could be to a significant reduction in their license values. After all, the existence of clear-channel AM signals from distant cities has been a great incentive for many in the U.S. and elsewhere (including FCC Chairman Pai) to become interested in radio as a career or avocation.

We agree with Henry Engineering that by proposing to change radio stations’ interference protection contours, the Commission is essentially proposing to change what amounts to the equivalent of a property boundary line.  What then would be the Commission’s strategy for compensating licensees for the interference protection that will be taken away by what essentially amounts to eminent domain?

In the realm of nighttime signals, we assert that the Commission's proposed Rule of protecting the 0.1 mV/m groundwave contour from co-channel signals (presumedly groundwaves) as stated does not provide adequate skywave protection criteria and will markedly harm the Class A stations' coverage at night. We suggest that for the time being, no change in the Class A nighttime protections be made until more detailed studies, including a refinement of protected contours and their associated D/U values, be completed. We further agree with Comments from Broadcast Transmission Services, LLC (BTS) that a more modern, numerical approach to AM coverage studies be adopted to secure more accurate, consistent  estimates of service contours and populations therein. We also assert that the 10% skywave predictions be included for Class A coverage calculations, which should retain the existing 25% exclusion methodology. Further, the suggestion by   BTS to provide a minimum D/U ratio of 40 dB for co-channel and 20 dB for adjacent-channel protection at a 2.0 mV/m contour for all stations is worthy of more detailed study before any permanent rules are adopted. We would much prefer a co-channel figure of 50 dB, as is realized by the current 0.5 mV/m contour with 26 dB D/U to further improve close-in coverage SNR, but these alternatives should be carefully weighed by the Commission before proceeding with this action.

For critical hours, we vigorously oppose the proposed elimination of all such protection for Class A stations. This is simply illogical, as it ignores the physics of the  ionosphere at AM frequencies within  2 hours of local sunset. This protection (co-channel and adjacent) should absolutely continue as currently, though perhaps to the 0.2-mV/m contour of the Class A station. This protection is especially important in periods of reduced solar activity, which permit much more skywave during early and late daytime hours (witness any Class C channel at those times!)

We understand the high cost of directional arrays for smaller stations, but reducing power at a given time (as currently done) is not a significant problem. Moving the protection contour on the Class A station could afford more power in critical hours to the smaller stations and is a useful operational compromise, though the use of synchronous boosters is a very valid alternative.

Change Nighttime RSS Calculation Methodology

We generally concur with many others in the engineering community that the nighttime RSS calculations need to be simplified, whether by returning to the traditional 50% exclusion method of RSS calculations of interfering signal strengths, or by retaining the 25% exclusions; in any event, more study is needed to fully resolve this issue. The calculations should be made on a site-to-site basis, which is much simpler and provides similar overall results to the traditional contour studies. However, due to the importance of minimizing the disturbing effects of spectral overlap by adjacent-channel signals, we believe that it would be counterproductive to achieving quality reception to ignore the adjacent-channel contributions entirely as many have suggested (from the perspective that AM will never be other than voice-grade). We therefore suggest as a compromise that adjacent-channel signal contributions be included into the RSS calculations, with a 50% weighting for each adjacent signal prior to the co-channel RSS computations using the 50% or 25% exclusion adjustments. This obviously favors the co-channel signals but permits a realistic accounting for stronger adjacent-channel signals in the mix that could materially degrade the desired signal.

Commission Proposal B –  Change Daytime Protection to Class B, C, and D Stations

This part is undoubtedly the most controversial of the proposed protection Rules changes. Although there is considerable pressure to downgrade the basic longstanding protection limits on Class B, C, and D, we note that, as we have stated earlier in the section on AM noise, the simple boosting of transmit powers, though helping the close-in signal-to-noise ratios, only does so at the expense of materially and permanently degrading fringe-area reception by replacing random and occasional impulse noise with full-time, wide-area, and pervasive co-channel and adjacent-channel interference. Actually, the correct, quickest, and most economic fix is for the Commission to strengthen and rigorously enforce its own noise Rules, including Parts 15 and 18.

Meanwhile, many Commenters seek a change to 2.0 mV as the protection contour, with a standard 26-dB D/U for CCI and a 0-dB D/U for ACI. The additional interference areas (correctly defined as zones where overlap causes D/U signal-strength ratios of less than 26 dB [i.e., a factor of 20], as dLR and others have explained, do in fact occur at the outer boundaries of the protected contour; these areas typically encompass only roughly 1/3  of the total contour-overlap area. At any rate, the interference zone also extends over the full sector (based on a single interferer) outside the protection contour, thus producing excessive (i.e., > 26 dB D/U) in those outer areas previously still inside the traditional 0.5 mV/m contour. The net result is that we have traded better SINR inside the 2.0-mV/m area for much more CCI (and ACI) outside.

If, ideally, all stations could simultaneously increase power by the same proportions, the relative CCI and ACI levels would not change; however, this can never in reality be the case, since (as has been noted earlier) not all stations can increase their operational power due to statutory limits (i.e., 50 kW max.), antenna siting issues, or basic fiscal constraints.

Under the current §73.37 AM allocation rules, assuming uniform ground conductivity, the minimum co-channel D/U ratio at the 0.5 mV/m groundwave contour is 26 dB.  At the 1.0 mV/m groundwave contour, the D/U ratio increases to 38 dB, and at the 2.0 mV/m contour, it rises to 50 dB.  As cited by Henry Engineering, under the currently proposed allocation scheme, the co-channel D/U ratio at the 0.5 mV/m contour would be just 2 dB; at the 2.0 mV/m contour it would be 26 dB.  The Commission’s recent proposal therefore represents a potential 24-dB degradation in the daytime co-channel signal to noise ratio.  This is a huge reduction in audio quality at a given signal strength contour.  At the 5.0 mV/m groundwave contour, the proposed D/U ratio would only be about 42 dB, hardly qualifying as a “city grade” quality signal.  If the co-channel protection groundwave contour is changed from 0.5 mV/m to 2.0 mV/m and the D/U ratio of 26 dB is unchanged, a given station could be subjected to as much as 24 dB more relative interference (D/U) at a given contour. In short, if the protection contour is changed, then the associated D/U ratio must be altered as well to maintain equilibrium.

As an illustration of the effects of these proposed contour changes, Appendices C, D, and E  contain notional plots of the scenarios involving a simple pair of stations, where station A on the left stays at its initial power, while station B upgrades to 4× its original level, with 1.0 mV/m contour protection, and then to 16× with a 2.0-mV/m contour. Note the significant CCI signal encroachment on the station A coverage area (the red crescent-shaped areas. Of course, real situations will involve more stations, usually in different orientations, and will result in more extensive overlap and, thus, CCI and ACI at multiple headings.

ADDITIONAL PROPOSALS FOR AM REVITALIZATION

KTL believes that the following significant steps can also be undertaken now to encourage revitalization of the AM radio service, and we strongly encourage the Commission to take them quickly. To accomplish this, the Commission must first move forcefully to enforce Part 15/18 Unintentional Radiator rules on Utilities and others, and further mandate major improvements in AM receiver performance, especially to achieve near-parity with FM. Without these key high-level actions,  many of the other suggestions for improving AM service will likely become moot unless the listening public is incentivized to return to the band, via the rapid establishment of noticeably better audio and reception conditions. These Further Proposals (presented also in the original NPRM)  are generally ordered in terms of overall importance to the sustainability of AM radio, with the first (concerning noise enforcement, presented earlier in this document) being considered by far the most critical. All of these concepts have been presented to the Commission and at NAB Conferences, as well as AFCCE and other groups. We believe these proposals are very significant to revitalizing AM radio, but have yet not received any official response from the Commission; it is our hope that these will be considered by the Commissioners and staff in the very near future.

Further Proposal 2  Immediately Open Local Synchronous Booster Stations to Permanent 	Licenses

We emphatically support the earlier 13-249 Comments of Eng. Wifredo G. Blanco-Pi on the beneficial use of synchronous transmission by AM stations to provide coverage of isolated areas of significant population, as he cited from his experience with multiple installations on the island of Puerto Rico. The requirement to continually re-authorize synchronous boosters is an unnecessary administrative burden for both the licensees and the Commission staff. The technology of local and wide-area synchronization of dispersed transmitters via GPS and similar means has been well-proven in numerous communications venues, including television, cell-phone base stations, and even in HD radio systems; thus, the technology is well-established and there is no need to continue its "experimental" designation. We fully agree with the Reply Comments on the subject in the 13-249 NPRM from dLR, and further strongly endorse the new Rules they have suggested for synchronous boosters. We cite these items below, with a few modifications based on our own studies:

(1)  A synchronous AM system should be defined as a master, licensed standalone  station with one or more synchronized, co-frequency, lower-power booster transmitters carrying identical modulation formats and time-synchronized audio signals. All boosters shall be sited within the 2 mV/m daytime contour, or 40 miles of the master transmitter's location, whichever is greater.

(2)  Synchronous operation shall require absolutely synchronized carrier frequencies (also see Further Proposal 4 below). If  precision offset operation is desired to minimize standing-wave fading zone effects between transmitters, this shall be accomplished via cyclic or randomized phase-shift means in the carrier reference of the booster unit(s).

(3)  Synchronous systems shall consist of multiple authorized transmitters with normally protected daytime signal-level contours that overlap or are contiguous with nighttime operation, even if higher nighttime interference levels might result in disjoint interference-free contours.

(4)  Nighttime-only synchronous transmitters at locations meeting the daytime criteria shall be authorized, if desired, so long as they comply with normal channel allocation Rules.

(5)  Each transmitter in a synchronous system should be studied for allocations with each such transmitter considered individually.

(6)  A system of synchronous transmitters, each of which meets all applicable allocations criteria with regard to protecting other stations (except each other) from interference when considered alone, shall be licensed without regard to extension of the coverage area of the primary station. If overall coverage is expanded without interference being produced to any other station, that is explicitly permitted.

(7)  As synchronous boosters may have intentionally limited power and coverage areas, no minimum antenna efficiency, height or ground system requirements shall apply to them.

(8)  A synchronous system of transmitters (i.e., a master station and its set of boosters) shall count as one station for the purposes of ownership rules, license renewal, and transfers.

The aforementioned synchronous-booster system could be of significant benefit to Class-C and -D stations with limited nighttime coverage, as well as other stations (mostly Class-B but also some Class-A) with deep nighttime directional-antenna nulls. All these stations could greatly benefit from the improved population coverage at night and during critical hours, particularly where urban/suburban sprawl has expanded beyond the stations' existing strong-signal zones. Unlike FM translators, such on-channel boosters would serve to increase the AM stations' audiences while concurrently maintaining the future viability of the band. The use of synchronous boosters could clearly provide new, productive  nighttime AM signals into each community from the local area, at very low cost to the stations involved and with significant public benefit. Further, these and other such synchronous boosters could well prove to be an economic boon to many struggling AM operations by permitting tailored coverage areas to match listening locales. In addition, the strategic use of synchronous boosters can provide better coverage to specific close-in areas (with large populations) for all classes of stations without causing significant increases in interference to other co-channel and adjacent-channel stations. This concept could particularly yield useful increases in local stations' nighttime coverage while avoiding concurrent degradation to distant existing Class A skywave service areas, as calculated by either 50% or 25% exclusion methods.

For synchronous applications, it is useful to examine how the phases/delays of the audio and RF components of the AM radio signals can affect reception quality in the field, particularly in signal-overlap regions. For instance, the RF signal delay is very roughly 1 millisecond for 186 miles (corresponding to the speed of light in air). At a point equidistant from two omni-directional, co-phased (synchronous) transmitters with equal power and propagating via groundwave mode over land paths of identical RF conductivity, the two RF signals will arrive with equal amplitudes and delays (phases). Now if we assume that the RF carriers and the sideband audio signals are precisely in phase (matched in time) as they leave the two antennas, at the exact midpoint between the two transmitters the RF signals and the detected audio will also be in phase; the signals can be added algebraically to calculate the resultant. Now for points not equidistant from the two transmitters, the RF signals will add vectorially; in general, there will be augmentations and cancellations of the two waves occurring at spatial intervals of one-half wavelength, essentially the same as is the case for standing waves on a mismatched transmission line. Modulation distortion will be minimal near the 0-additive points and rise somewhat at quadrature-phase contours, and peak as the summed signal approaches null at the 180 points. Obviously, near the equal-signal points, the standing wave patterns will exhibit maximum variations; in fact, §73.182(t) of the Commission's Rules defines the region of “satisfactory service” for synchronous stations as areas where the ratio of field strengths is ≥ 6 dB (≥ 2:1). However, the Rules as quoted did not assume the accurate time-synchronization of both audio components; as cited by Blanco-Pi and dLR in earlier 13-249 Comments, the audio time-matching significantly mitigates the apparent distortion and reduces the area of discernible distortion. The interference patterning in the synchronous overlap zone can be further reduced by phase-dithering of the booster signal(s), either in a cyclic or random-phase fashion. Terrain variations, buildings, and other groundwave scatterers or diffractors (i.e., multipath sources) will also reduce the magnitude of these overlap-zone disturbances via the inherent dithering of carrier phase. In moving vehicles, the audible effects will be even less, especially on speech programming. It has been long known [4] that the distortion zones can be designed to fall over less-populated areas and major arteries; for instance, the overlap zones (near 1:1 signal ratios) would obviously be configured to fall in the more rural areas between cities. Further, U.S. Patent 7,881,416 describes the further reduction of these standing-wave patterns (and distortion) with the use of additional low-power localized boosters in or near the equal-signal zones and strategic phase-dithering. The net result of all this is that synchronized AM boosters are indeed ready for immediate wide-scale deployment.

Further Proposal 3  Mandate Regional/National Synchronization of All AM Stations

Three papers, published by the IEEE and NAB in the 2007-2010 time frame, [4], [5], [6], four U.S. Patents [7], and a paper at NAB 2015 [8], described a straightforward but highly accurate carrier-frequency synchronization scheme for actively, automatically locking multiple, remotely located AM broadcast transmitters to a common frequency/timing reference source such as GPS. The extremely tight frequency lock (to ~1 part in 109 or better) permits the effective elimination of audible and even sub-audible beats between the local (desired) station’s carrier signal and the distant stations’ carriers. Generally, an AM radio listener during the evening and nighttime hours, and to a lesser extent in the early morning, receives undesired skywave signals from several distant co-channel stations as well as the desired local (groundwave) signal. These carrier-beat components in the current (non-synchronized) scenario can cause annoying modulations of the desired station’s audio at the receiver and concurrent distortion of the audio modulation from the distant station(s) and often cause listeners to “tune out” due to the poor reception quality. This is quite understandable since the average carrier power is on the order of 10 dB above that of the typical levels of the sideband modulation components, and the inter-carrier beats will dominate the receiver's AGC and thus modulate the audio level. Along with EMI, these beat-related effects are certainly a (if not the) principal factor in the degradation of evening and nighttime AM fringe-area reception quality and the resulting loss of outlying listeners for virtually all AM stations. Perhaps the most deleterious aspect of these beats is the listener-annoyance factor, in that the high-level artifacts (volume modulation, cyclic distortion, and pronounced swishing sounds) often quickly induce listener tune-outs. This situation is not only progressively worse further into the fringe areas of the desired stations (usually in the outer suburbs of the city of license), but also occurs much closer in, in the deep nighttime nulls of directional stations. The current poor state of repair of many AM directional arrays, plus the low-power pre-sunrise/post-sunset (PSRA/PSSA) operations at many Class-D stations, only exacerbates these problems.

If, however, we employ carrier synchronization, all of these signals' frequencies can be held to within about 0.01-0.001 Hz of each other, and any resulting carrier beats will be of such long periods that the beats will be effectively suppressed by the action of the receiver’s AGC circuitry and become completely unnoticeable to the listener. The significant reduction or elimination of the beats and related effects achievable via synchronization will greatly enlarge the effective co-channel interference-limited listening area of the desired station (from 4 to 10 times as indicated in our tests, dependent on program material) and simultaneously reduce the corresponding interference of the local transmitter to the distant stations as well. In addition, AM stereo (CQUAM) reception will be particularly improved by minimizing the phase shifts induced by co-channel interfering signals; HD or all-digital signals will also benefit via reduction in beats from co-channel analog signals.

The automatic frequency-control hardware described in the references is inexpensive, requires no periodic recalibration, has essentially zero long-term drift, and could employ alternate wide-area frequency references of suitable accuracy, including broadcasts from WWVB, LORAN-C, and equivalent sources. The basic configuration of a commercially available GPS-disciplined oscillator which solves this problem is extremely simple and costs under $300 (including the GPS antenna). The main oscillator is a conventional high-stability ovenized quartz-crystal type. To counter long-term drifts, the oscillator is automatically adjusted to track a high-precision source of standard frequency obtained from a specialized GPS receiver (or other source), usually at 10.000 MHz. This very stable local reference frequency is then used as a clock for a standard digitally implemented frequency synthesizer, which is programmed to generate the specific (AM broadcast) transmitter carrier frequency desired. The stability of the disciplining source, typically ~ 1 part in 109 to 1011, is thus transferred to the final AM transmitter carrier output frequency. Most modern, synthesizer-based transmitters can directly lock to the precision disciplined 10-MHz source, while older units usually require references at either 1×, 2×, or 4× the final frequency. In these latter cases, the existing transmitter crystal can usually be satisfactorily “pulled” via injection locking.

The effectiveness of the synchronization concept to reduce interference effects was demonstrated by ORNL researchers in a laboratory test setup, as described in the references above. Many hours of careful subjective listening were conducted, with the two interfering units both precisely on-frequency with the main unit (synchronous operation) and with the two interferers at various frequency offsets, from below 1 Hz to above 10 Hz. The most audibly annoying beats were generally judged to be below roughly 2 Hz, so several tests were conducted with offsets of 0.7 and 1.7 Hz, respectively, which tend to more closely emulate current AM channel beat characteristics. Subjective measurements to determine the familiar audible interference assessment criteria of “imperceptible”, “perceptible”, “annoying”, and “objectionable” were made and documented. Overall, the net effect to the listener of synchronizing the AM carriers and thereby eliminating the beats is on average about 6 dB minimum and can often be as great as 10 dB; this is of major importance in evening, nighttime, and pre-sunrise situations where the SIR due to incoming skywave signals can degrade to levels of 12 dB or worse. From the standard propagation data, at the nominal fringe signal level of 0.5 mV/m (for all Classes of stations except A, defined as 0.1 mV/m), the daytime, groundwave co-channel signals (re §73.182) must each be no more than 1/20 the amplitude (−26 dB) at the stated field-strength contour [or 25μv/m, (5μv/m for A)]. The same corresponding nighttime values of acceptable co-channel interference levels (−26 dB) are specified for Class A, 0.5 mV/m (50% skywave) contours and the 2 mV/m contours for Class B (groundwave). Allowing for finite ground conductivities, it is evident that an improvement of 6 dB in effective co-channel levels will nearly double the interference-limited contours of the stations compared with the standard, non-synchronous case (almost quadrupling the equivalent coverage area). As will be described later, our simulations with real broadcast audio demonstrated that for some types of programming (i.e., with good masking properties) the effective improvement can even approach 10 dB, which could nearly triple the interference-limited coverage range! With the beats eliminated, the background audio from the co-channel stations will be clean; often, the so-called “cocktail party” effect will reduce the apparent level of those signals to the listener even further, especially in high-background ambients such as automobiles. The net result of these effects will be universally evident but particularly beneficial to nighttime operations at local Class-C and Class-D stations, whose coverage areas are already acutely curtailed by heavy co-channel skywave interference. For these latter classes, the near-quadrupling of equivalent coverage at night should be a major benefit, particularly to listeners in outlying suburban areas.

The principal drawback to the approach is a practical implementation issue ⎯ all stations on the channel in question (at least those with signals above the noise floor at the receiver) must be closely frequency-locked to a common precise reference as just described, or the beats will not be eliminated. It is therefore incumbent on the Commission to mandate the wide-area synchronization requirement for all AM stations as soon as practicable. In our view, wide-area AM transmitter synchronization is (and at very low cost) the only technology that, when adopted, will immediately benefit all stations, all frequencies, and all receivers, both day and night

Further Proposal 4  Encourage Establishment of Minimum AM Receiver Performance 	Levels to Provide Parity with FM-Band Counterparts

As previously covered in the Overview section of this document and in a recent paper [8], for the long-term health of the AM radio medium, it is absolutely essential that very close to full parity be established for new AM radio receivers versus their FM radio counterparts. We appreciate that the Commission currently lacks statutory authority to enforce minimum specifications for AM receivers marketed in the U.S., but as suggested by Commission staff, the FCC Lab could certify desirable performance levels, with a "seal of approval" for complying units, to assist in substantive improvements in AM radio receivers for the American public.

This includes, as was cited earlier, all key AM receiver performance attributes, including: (1) low internal noise floor, well below the average AM-band atmospheric noise level; (2) high overall RF sensitivity, selectivity, and dynamic range, to provide adequate amplification of weak signals, even in the presence of significant adjacent- and/or alternate-channel signals, especially in strong-signal environments; (3) highly effective noise (EMI) rejection, including staged RF and IF noise blanking, accompanied by appropriate audio blanking/expansion when required; (4) full 10-kHz audio bandwidth capability with low detector distortion, plus dynamic bandwidth control (including adaptive 10-kHz notch filtering) as dictated by noise and adjacent-channel interference; and (5) stereo capability (if the receiver has FM stereo capability, it must have CQUAM decoding for AM). Without the first three requirements (this also includes the associated AM antennas), basic AM reception will suffer greatly compared with FM; without the last two, the output sound quality cannot be closely competitive with FM (10 kHz full bandwidth on AM versus 15 kHz maximum for FM).

Several Commenters in the 13-249 Proceeding have suggested a 6-kHz bandwidth for AM, largely to minimize potential ACI to next-channel stations, based on earlier data obtained during NRSC tests on typical AM receivers. Although this might be adequate for all-speech programming, for many small- and medium-market stations, who program music for significant parts of their broadcast days,  the full 10 kHz bandwidth is definitely needed. The line of reasoning used by some to back a 6-kHz bandwidth was largely circular in nature; the use of wider bandwidths (again, we believe 10 kHz is truly needed for music) was not preferred on average by the test listeners simply due to the high ambient channel noise levels. Actually, if the noise for these tests were processed out correctly via proper dynamic filters and blanking circuits, a greater bandwidth would have certainly been favored, since it is well known that people in general prefer wider audio bandwidths, but only if they are essentially noise-free. To eliminate all audio content above 6 kHz using the current sorry state of AM receivers as justification is to ignore the fundamental need of AM to achieve a close parity with FM sound quality in order to remain sustainable in the future. As mentioned previously, older receivers are so bad that such ACI concerns are probably moot, and the new, high-performance units can adaptively narrow detection bandwidth as needed in fringe-reception or high-noise scenarios. On top of all this, so poor is the AM sound of many current radios that to compensate for the overwhelming lack of treble (with a typical 2.5-kHz bandwidth), the bass is likewise reduced via low-cut filters to "balance" the sound, rendering the AM audio truly anemic (muffled and tinny) compared with FM (rich and full, and in stereo to boot). This stunning disparity cannot continue if AM broadcasting is to be truly revitalized.

Further Proposal 5   Revise Allocations for Improved Utilization Of AM Expanded Band

We agree with dLR, H&D, and others that the establishment of standardized allocation rules for use in the AM Expanded Band – the channel frequencies from 1610 through 1700 kHz – are timely. As we stated in our Reply Comments in the original 13-249 NPRM action, the existing rigid, inefficient allocation guidelines clearly stifle the opportunity for standard-band AM stations to voluntarily migrate to these frequencies and also restricts the flexibility of stations already there to make facility changes. As is, the Expanded Band is comparatively under-developed and should be seamlessly integrated with the rest of the AM service, including the adoption by the Commission of a set of allocation rules fully consistent with those in the standard band. The more efficient use of these frequencies would definitely aid in reducing the congestion in the standard band; in addition, the addition of at least one or even two Class-C channels in the band would be a boon for small communities without an existing local AM outlet. It would also seem logical for the Commission to reclaim the 1610-kHz frequency from NTIA, move all the TIS highway-information stations to 530 kHz (with a concurrent RF power increase for them to overcome the higher levels of noise, both atmospheric and man-made, at that lower frequency). This would better serve the public by making TIS more consistent throughout the U.S. Also, TIS could be further utilized in local or wide-area emergency scenarios to alert and inform mobile listeners. A TIS alerting-tone system could then be added at low cost to provide timely traffic information and other forms of emergency communications.

Another future possibility would be to introduce potential all-digital operational experiments into the Expanded Band, owing to the much-reduced level of impulse noise at those frequencies and the comparative lack of skywave interference versus the standard band. However, we do not believe that these digital operations should in any way impede the growth of analog AM operations, and all future digital AM-band transmissions must be required to properly integrate with existing analog AM signals, without causing additional interference issues.

Conclusions

As we stated in response to the original 13-249 NPRM over five  years ago, AM radio is a longstanding American institution, a source of unique voices, and one that we can ill afford to abandon, particularly in light of its unique groundwave and nighttime skywave propagation characteristics and tremendous reach, especially in times of local, regional, and even national emergencies. During the recent national disasters, Hurricane Katrina and Hurricane Sandy, AM radio stations proved to be the news source that the public utilized more than any other when telecom and other services were unavailable.  In addition the Primary Entry Point Network operated by the Department of Homeland Security and the Federal Emergency Management Agency to facilitate Presidential access to the US population in the event of a national emergency is primarily comprised of AM 50-kW clear channel stations Truly, this AM Revitalization action has the rare  potential of conserving a unique national resource.

We believe that AM radio stations can be relied upon to provide needed service well into the future, but the Commission must take several bold steps in the very near future to preserve AM radio for future generations of Americans. KTL believes that the suggested actions can be undertaken rapidly to encourage a general revitalization of the AM radio service, and we strongly encourage the Commission to take them now. We reiterate our agreement in principle with many of the Further Comments and Reply Comments already offered by others in the consulting engineering community, though with some alternative suggestions. Our proposals are driven by our overriding view that to save and revitalize the AM band for broadcasters and the public, the Commission must move rapidly and forcefully to enforce Part 15 and 18 Unintentional Radiator rules on Utilities and others, enforce Part 15 regulations on non-compliant imported electronics via actions against their domestic vendors and further encourage major improvements in AM receiver performance, especially to achieve near-parity with FM. Also included in our proposals to improve AM reception are the simplified adoption of synchronous booster stations to augment existing AM station coverage and the mandate of wide-area GPS-based synchronization to significantly reduce co-channel interference via the elimination of carrier beats. It is also essential that, as per Commissioner O'Rielly's statement, lacking a robust engineering consensus concerning the instant proposed major allocation Rules changes, the Commission defer any action until a high level of general agreement is achieved. Without these high-level actions, most of the other suggestions for improving AM service offered by our firm and other Commenters will likely become moot unless the listening public is incentivized to return to the band, via the rapid establishment of noticeably better audio and reception conditions throughout the U.S. The final, inevitable consequence of all this, if not addressed by the Commission,  will be the financial failure of the all too many struggling AM stations and the and the loss of service to the American public.

| Respectfully Submitted, Thomas F. King, M.S.EE, President Stephen F. Smith, Ph.D.EE, Consultant Kintronic Laboratories, Inc. Manufacturers & Consulting Engineers 144 Pleasant Grove Road Bluff City, TN 37618 Phone: (423) 878-3141 E-mail: tking@kintronic.com | January 22, 2019 |
| --- | --- |

References

[1]  NRSC Reference Library Document No. NRSC-R13, “AM Technical Assignment Criteria: An Examination of Issues Raised in MM Docket No. 87-267”

[2] Appendix III,AM Radio Interference Study, Final Report, MM Docket No. 87-267, June 1988.  B. Angell & Associates, Chicago, Illinois.  Prepared for the National Association of Broadcasters, Washington, D.C.   https://www.nrscstandards.org/reports/nrsc-r14.pdf

[3] Kintronic Laboratories, Inc. Reply Comments in FCC Proceeding 13-249 on AM Revitalization,  March 20, 2014.

[4] All-Channel Receiver Act, Wikipedia article, http://en.wikipedia.org/wiki/All-Channel_Receiver_Act.

[5] Stephen F. Smith and James A. Moore, “A Precision, Low-Cost GPS-Based Synchronization   Scheme for Improved AM Reception”, IEEE 2006 Broadcast Technical Symposium, Washington, DC, September 29, 2006.

[6] Stephen F. Smith, James A. Moore, and David W. Allan, “A Precision, Low-Cost GPS-Based Synchronization Scheme for Improved AM Reception”, National Association of Broadcasters Technical Conference, Las Vegas, NV, April 15, 2007.

[7] Stephen F. Smith and James A. Moore, “A Precision, Low-Cost GPS-Based Transmitter Synchronization Scheme for Improved AM Reception”, IEEE Transactions on Broadcasting, Vol. 55, No. 1, March 2009, pp. 71-78.

[8]  U.S. Patents 7,881,416; 7,587,017; 7,218,696; and 6,563,893, to S. F. Smith and J. A. Moore.

[9]  Stephen F. Smith and Thomas F. King, "Smart AM Receivers for the 21st Century", Proceedings of the National Association of Broadcasters Engineering Conference, Las Vegas, NV, April 12, 2015.

[10] Thomas F. King, Stephen F. Smith, Wifredo G. Blanco-Pi and Jorge G. Blanco-Galdo, "Field Trial Results of AM Transmitter Carrier Synchronization", Proceedings of the National Association of Broadcasters Engineering Conference, Las Vegas, NV, April 12, 2015.

APPENDIX A

APPENDIX B

APPENDIX C

APPENDIX D

APPENDIX E