IWG-3/049 (7.27.18)

# IWG-3 suggested edits to RCS draft Proposal on WRC-19 Agenda item 1.3

# UNITED STATES OF AMERICA

# DRAFT PROPOSAL FOR WRC-19

Agenda Item 1.3:  to consider possible upgrading of the secondary allocation to the meteorological-satellite service (space-to-Earth) to primary status and a possible primary allocation to the Earth exploration-satellite service (space-to-Earth) in the frequency band 460-470 MHz, in accordance with Resolution 766 (WRC-15)

BACKGROUND:  The 460-470 MHz frequency band is allocated on a primary basis to the fixed and mobile services.  The meteorological-satellite service currently has a secondary allocation in this band.  Under No. 5.289, “Earth exploration-satellite service applications, other than the meteorological-satellite service, may also be used in the bands 460-470 MHz for space-to-Earth transmissions subject to not causing harmful interference to stations operating in accordance with the Table”.

Within this frequency band the Argos Data Collection System (ADCS) monitors more than 21,000 active Argos platforms collecting data for over 2,000 distinct projects in 100+ countries.  The administration of the Argos program is under a joint agreement between the National Oceanic and Atmospheric Administration (NOAA) within the United States and the French Space Agency, Centre National d’Etudes Spatiales (CNES).  Additional partners include the European Organization for the Exploitation of Meteorological Satellites (EUMETSAT), and the Indian Space Research Organization (ISRO).

Critical applications of the ADCS include atmospheric and ocean monitoring/research, tropical cyclone forecasting, fishery management, oil spill tracking, fishing vessel tracking, search and rescue modeling (at sea), anti-piracy alerting, import/export and hazardous materials tracking, endangered species studies, migration mapping, and wildlife tracking and management.

The meteorological-satellite (space-to-Earth) service operates on a secondary basis relative to the fixed and mobile services and thus it must not interfere with these services.  To protect the fixed and land mobile services within the United States, a power flux density (pfd) of -152 dB(W/(m2∙4kHz)) has been imposed on the meteorological-satellite (space-to-Earth) service.

In accordance with Resolution 766 (WRC-15), the ITU-R has conducted sharing studies to ensure the protection of incumbent services and to develop a pfd limit that will protect incumbent services globally from potential interference in the frequency band 460 – 470 MHz.

Studies have demonstrated that sharing is possible between meteorological-satellite (space-to-Earth)/earth-exploration-satellite (space-to-Earth) services and the incumbent services in the 460 – 470 MHz frequency band if the pfd limits proposed below are applied.  Based on the results of sharing studies, this proposal supports an allocation upgrade from secondary to a primary for the meteorological-satellite service (space-to-Earth) and a new primary allocation to the earth-exploration-satellite (space-to-Earth) service in the frequency band 460 – 470 MHz band.  This proposal applies a set of elevation angle dependent pfd limits to the meteorological-satellite and earth exploration-satellite services in order to protect the incumbent services globally.

Proposal:

ARTICLE  5

Frequency allocations

Section IV –  Table of Frequency Allocations
(See No. 2.1)

MOD  	USA/AI 1.3/1

460-470 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 460-470 | FIXED MOBILE 5.286AA METEOROLOGICAL-SATELLITE (space-to-Earth) EARTH-EXPLORATION-SATELLITE (space-to-Earth) 5.287 5.288 5.290 ADD 5.A103 ADD 5.B103 ADD 5.C103 |  |

Reason:  The upgrade from the secondary MetSat and EESS allocations to primary will bring regulatory stability and certainty to the space agencies involved in Satellite Data Collection Programs.  With this upgrade, inclusion of this band in footnote 5.289 is no longer necessary.  Also, this upgraded allocation will ease coordination effort for Administrations.

.

ADD	USA/AI 1.3/2

5.A103	  In the frequency band 460-470 MHz, earth stations in the meteorological-satellite (space-to-Earth) and earth-exploration-satellite (space-to-Earth) services shall not claim protection from stations of the fixed and mobile services.

Reasons:	The incumbent fixed and mobile allocations maintain a higher regulatory status over the primary meteorological-satellite (space-to-Earth) and earth exploration-satellite (space-to-Earth) services by not being required to protect the metrological-satellite and earth-exploration satellite services.

ADD	USA/AI 1.3/3

5.B103	  In the frequency band 460-470 MHz, in order to protect systems of the fixed and mobile services, the power flux-density at the Earth’s surface produced by stations in the meteorological-satellite (space-to-Earth) and earth exploration-satellite (space-to-Earth) services shall comply with the  limits listed below under assumed free-space propagation conditions for all methods of modulation:

- For non-GSO spacecraft:

2)   For GSO spacecraft:

where   is the angle of arrival above the horizontal plane, in degrees.  (WRC-19)

JW comments: Note that Art 21 has the following text: “The power flux-density at the Earth’s surface produced by emissions from a space station obtained under assumed free-space propagation conditions for all methods of modulation.

Reasons:  The meteorological-satellite (space-to-Earth) and earth exploration-satellite (space-to-Earth) services shall protect the incumbent fixed and mobile services by restricting their operations according to these pfd limits.

ADD	USA/AI 1.3/4

5.C103  In the frequency band 460-470 MHz, stations in the earth exploration-satellite service (space-to-Earth) shall not cause harmful interference to stations in the meteorological-satellite service (space-to-Earth).  (WRC-19)

Reasons:  Maintain the current priority that the meteorological-satellite service (space-to-Earth) has relative to the earth exploration-satellite (space-to-Earth) service.

MOD  	USA/AI 1.3/5

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 690-1 700 METEOROLOGICAL AIDS METEOROLOGICAL-SATELLITE (space-to-Earth) Fixed Mobile except aeronautical mobile MOD 5.289 5.341 5.382 | 1 690-1 700 METEOROLOGICAL AIDS METEOROLOGICAL-SATELLITE (space-to-Earth) MOD5.289 5.341 5.381 |  |
| 1 700-1 710 FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile MOD5.289 5.341 |  | 1 700-1 710 FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile MOD5.289 5.341 5.384 |

MOD 	USA/AI 1.3/6

5.289  Earth exploration-satellite service applications, other than the meteorological-satellite service, may also be used in the band 1 690-1 710 MHz for space-to-Earth transmissions subject to not causing harmful interference to stations operating in accordance with the Table.

APPENDIX 7 (REV.WRC-15)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

ANNEX 7

System parameters and predetermined coordination distances for determination of the coordination area around an earth station

# 3	Horizon antenna gain for a receiving earth station with respect to a transmitting earth station

MOD	USA/AI 1.3/7

TABLE 8a     (Rev.WRC-19)

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | Space operation, space research | Meteoro-logical- satellite, mobile-satellite |  | Space research | Space research, space operation | Space operation | Mobile-satellite | Meteoro-logical-satellite | Mobile-satellite | Space research | Space operation |  | Broad-casting- satellite | Mobile-satellite | Broadcasting- satellite (DAB) | Mobile-satellite, land-mobile satellite, maritime mobile-satellite |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (MHz) |  |  | 137-138 | 137-138 |  | 143.6-143.65 | 174-184 | 163-167 272-273 5 | 335.4-399.9 | 400.15-401 | 400.15-401 | 400.15-401 | 401-402 |  | 620-790 | 856-890 | 1 452-1 492 | 1 518-1 530 1 555-1 559 2 160-2 200 1 |  |
| Transmitting terrestrial service designations |  |  | Fixed, mobile | Fixed, mobile |  | Fixed, mobile, radio-location | Fixed, mobile, broad-casting | Fixed, mobile | Fixed, mobile | Meteoro-logical aids | Meteoro- logical aids | Meteoro-logical aids | Meteoro-logical aids, fixed, mobile |  | Fixed, mobile, broad-casting | Fixed, mobile, broad casting | Fixed, mobile, broadcasting | Fixed, mobile |  |
| Method to be used |  |  | § 2.1 | § 2.1 |  | § 2.1 | § 2.1 | § 2.1 | § 1.4.6 | § 1.4.6 | § 1.4.6 | – | § 2.1 |  | § 1.4.5 | § 1.4.6 | § 1.4.5 | § 1.4.6 |  |
| Modulation at earth station 2 |  |  | N |  |  | N |  | N |  |  |  | N | N |  |  |  | N | N |  |
| Earth station interference parameters and criteria | p0 (%) |  | 0.1 |  |  | 0.1 |  | 1.0 |  | 0.012 |  | 0.1 | 0.1 |  |  |  |  | 10 |  |
|  | n |  | 2 |  |  | 2 |  | 1 |  | 1 |  | 2 | 2 |  |  |  |  | 1 |  |
|  | p (%) |  | 0.05 |  |  | 0.05 |  | 1.0 |  | 0.012 |  | 0.05 | 0.05 |  |  |  |  | 10 |  |
|  | NL (dB) |  | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  |  |  |  | 0 |  |
|  | Ms (dB) |  | 1 |  |  | 1 |  | 1 |  | 4.3 |  | 1 | 1 |  |  |  |  | 1 |  |
|  | W (dB) |  | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  |  |  |  | 0 |  |
| Terrestrial station parameters | E (dBW) in B 3 | A | – |  |  | – |  | 15 |  |  |  | – | – |  |  |  | 38 | 37 4 |  |
|  |  | N | – |  |  | – |  | 15 |  |  |  | – | – |  |  |  | 38 | 37 |  |
|  | Pt (dBW) in B | A | – |  |  | – |  | –1 |  |  |  | – | – |  |  |  | 3 | 0 |  |
|  |  | N | – |  |  | – |  | –1 |  |  |  | – | – |  |  |  | 3 | 0 |  |
|  | Gx (dBi) |  | – |  |  | – |  | 16 |  |  |  | – | – |  |  |  | 35 | 37 |  |
| Reference bandwidth | B (Hz) |  | 1 |  |  | 1 |  | 103 |  | 177.5 × 103 |  | 1 | 1 |  |  |  | 25 × 103 | 4 × 103 |  |
| Permissible interference power | Pr( p) (dBW) in B |  | −199 |  |  | −199 |  | −173 |  | −148 |  | −208 | −208 |  |  |  |  | −176 |  |
| 1 In the band 2 160-2 200 MHz, the terrestrial station parameters of line-of-sight radio-relay systems have been used. If an administration believes that, in this band transhorizon systems need to be considered, the parameters associated with the frequency band 2 500-2 690 MHz may be used to determine the coordination area. 2 A: analogue modulation; N: digital modulation. 3 E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth. 4 This value is reduced from the nominal value of 50 dBW for the purposes of determination of coordination area, recognizing the low probability of high power emissions falling fully within the relatively narrow bandwidth of the earth station. 5 The fixed-service parameters provided in the column for 163-167 MHz and 272-273 MHz are only applicable to the band 163-167 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reason: Consequential changes to AP 7 to remove the Meteorological Satellite Service given the proposed 5.A103.

SUP	USA/AI 1.3/8

RESOLUTION 766 (WRC-15)

Consideration of possible upgrading of the secondary allocation to the meteorological-satellite service (space-to-Earth) to primary 
status and a primary allocation to the Earth exploration-
satellite service (space-to-Earth) in the 
frequency band 460-470 MHz

# Reasons: Sharing studies have shown that an upgrade to a primary allocation for meteorological-satellite service (space-to-Earth) and earth-exploration satellite service (space-to-Earth) are compatible with the incumbent services and there is no further need for this resolution.

__________