Reply comments To Public Notice by:

Wireless Telecommunications Beurau, International Bureau, Office of Engineering and Technology, And Office of Economics and Analytics Seek Focused Additioinal Comment in 3.7-4.2 GHz Band Proceeding.

in GN Docket #18-22, RM-11791, RM-11778

By: Arthur Reis, CPBE, RadioArt Enterprises, LLC

First, a few credentials:

I am a Broadcast Engineer with nearly 50 years experience in the Industry, 44 of them as a Chief Engineer or Contract Engineer. I have been involved in broadcast services via satellite since 1981, first as a local CE at a long-form satellite-programmed affiliate, then for seven years as CE of the Chicago Studios of the Satellite Music Network. Since then, I have been a CE or a Contract Engineer (consultant) for a number of radio stations which receive programming via Satellite. I deal with both C- Band and Ku-Band systems. Both bands have their own unique properties.

I am also well acquainted with the advantages and the disadvantage of both short- and long-form programming delivery to stations via the Internet. And, I have some definite opinions with regard to the effort to further allocate the C-Band Satellite Services to additional use for carriage of G-5 Internet traffic and services.

Up front, I am against this proposed takeover, which is what it amounts to, of the Satellite Services C-Band for G-5, or for any other competing services. Here are the reasons why:

  - Satellite transmission, though highly reliable, is a very fragile communications medium. Signals coming down from satellites are comparatively weak to begin with, requiring a sizable antenna to enable a tight window of reception as well as proper signal gain, a low-noise block downconverter (LNB), and a receiver which, after all that, must be sensitive enough to receive the output of the LNB reliably. The current digital modes in use, or for that matter any digital modes, are quite sensitive to interference from whatever source. We out here in the hinterlands have learned that lesson the hard way, especially in HD radio, particularly, though not exclusively, on AM. Thus, we have before us an entity which in effect will completely 'take over' the C-band Satellite service by invasion, if this Proposal is adopted. This is not hyperbole. The C-Band Satellite Service has already had to deal with this issue by virtue of its sharing of the spectrum with the Long Lines Telephone System (which was, to be truthful, the first users of the spectrum, long before the advent of Satellite 'anything')'. C-Band Satellite, through the judicious  use of 10 MHz offsets, filters to help alleviate such interference to both services by each other, and careful planning by both, kept the problems at bay. But the glory days of long-lines over-the-air  transmissions are past, that service still has a presence on that spectrum nation-wide. Are their needs being kept in mind by this proposal as well. Long Lines and Satellite Long-Form services at least have learned to 'play nice' with each other by virtue of their long-standing cooperation; can the 'new bully on the block' with the voracious and rapacious spectrum appetite be trusted to be the same way? As you can see by my language with the last sentence, I doubt it. No, I really doubt it!

  - Let's assume that, by 'virtue' of this Docket, should it be adopted, that the G-5 interests get the power to occupy on the same spectrum as the C-Band Satellite Service. What are the alternatives to the C-Band service users? First, there's Ku Band, but there are two problems with that. First, if this Docket were codified into the Rules, it would set a precedent that would (not could; I've seen this scenario before)  eventuate in the Ku-Band becoming a future target for another such takeover. This would be catastrophic to Broadcasting, especially to television, which uses both C- and Ku Band satellite a lot. Actually, mostly. The loss of C-Band for television would be as, or even more catastrophic for them. And don't think that it can't happen. The appetite for spectrum by the wireless industry is overwhelming, due to the incredible public demand for their services, since the public has no clue as to what all this demand means. Nor should it have to. Still, the loss of the use of the C-Band itself has dire consequences.

- When I was employed at Satellite Music Network, which was and still is a C-Band spectrum user, a number of our affiliates, located in the Texas Gulf area, were once inundated by a hurricane. Those stations were able to put all of their human resources into dealing with the gathering of information regarding the hurricane and its consequences, simply by letting Satellite Music Network 'do' the regular programming, and pre-empting every commercial break period with news and information needed by the public. Information was compiled between breaks and then disseminated. Pre-emption of the Satellite Service was done as needed. That was made possible by the fact that the C-Band was used for the satellite-provided program. SMN's signals were still receivable at each station. This is not possible with Ku-Band Satellite. Ku-Band, by virtue of the frequency of its signal, is nowhere near as robust as is C-band in heavy weather. Indeed, a moderate thunderstorm in the area of any receive dish will take the signal out. And no Ku-Band signal will work in a hurricane. Rain fade at Ku-Band frequencies is real, it can't be wished away, it even affects Signals on Studio to Transmitter links operating there, and thus it must yield to C-Band signals in that regard.

  - I understand that there is considerable thought developing at the FCC regarding replacement of satellite-based programming, with the Internet. As a contract Engineer, I have a number of clients who use both Satellite and Internet for their long-form program delivery. In almost all cases, the Satellite is the predominant means of delivery, with Internet as secondary. In one case, Internet is the main form of delivery, with Satellite secondary. In two more cases, the station gets all of its long-form program delivery via the Internet. From what I (and many others who do what I do) have learned from our experiences of the differences between the two methods, Satellite is far and away the more reliable method of transmitting long-form programming. Only twice have I see it fail. The failure of Galaxy IV in 1998 is of course a legend. A major power outage at the Chicago WGN TV uplink in the late 1980's was another, though that wasn't a satellite issue but affected millions of viewers and listeners of that uplink's transmissions.

  - On the other hand, unlike what too many people believe, the Internet is nowhere near ready for  'prime time' where long-form programming, either radio or television, is concerned.  At the present state of the technology, Failures of many types abound. Bandwidth changes result in audio anomalies. Latency changes from time to time affects service. The differences in quality from urban to rural services abound. Outright loss of service from on-line outages are not uncommon. The advent and proliferation of fiber optics into the Internet mix has alleviated but not eliminated many of these problems. But fiber is nowhere near everywhere yet. And as the demand for services grows, there's a limit even to what fiber can handle. And it isn't available for the mobile market, only for interconnection and in fixed locations. There may come a time when the proliferation of fiber and other terrestrial high-speed throughput options becomes a truly viable option for broadcasters using long-form programs, both for radio and television. But, in my estimation, that time is still a long way, and a lot of money, off. And that is an important consideration.

  - My final point is regarding a non-technical aspect of the implementation of this proposed Docket. The concept of a wire-based delivery system, as opposed to a satellite-based system, also involves choice---choice of what we as a people listen to, and what we as a people watch on a flat screen. We as a People have already seen what censorship is like within social media. It's all over the news. Add to that the fact that countries which are ruled by dictators, or wherever there is 'top down' centralized control, has moved their communications entirely to a wire-based system. The whole country is wired for the functions which used to be the province of radio and television. With a wired-based system, governments, or pseudo-governments (read that, Facebook, YouTube, and Twitter) can much more easily control the flow of information, and punish or ban thought with which said government or private entity doesn't agree, or finds offensive in any sense, or considers to be subversive. The free flow of information and thought requires more that just one form of delivery. Radio, Satellite and yes, wired-based delivery systems, used together, form the best solution to potential tyrrany, and allows us to be better free as a country. The surrendering of spectrum designed to maintain the freedom, to those entities which has the greater potential of taking it away, should be considered reprehensible to a free people. To those of us who are Constitutional Conservatives, it already is.

Let those who would seek to take away satellite spectrum and replace it with their own technology, go somewhere else.

Sincerely,

Arthur Reis