Mr. Tom Sullivan

Chief, International Bureau

Federal Communications Commission

445 12th Street, SW

Washington, DC  20554

Dear Mr. Sullivan:

The National Telecommunications and Information Administration (NTIA), on behalf of the Executive Branch agencies, approves the release of the enclosed proposals for the 2019 World Radiocommunication Conference (WRC-19) which address:

- Agenda Item 1.16 – Radio Local Area Networks studies at 5 150-5 925 MHz;

- Agenda Item 9, Issue 9.1.5 - Incorporation of Recommendations M.1638-1 & M.1849-1 in Nos. 5.447F & 5.450A by reference; and

- Agenda Item 10 – Earth Exploration-Satellite Service allocation in the frequency range 40-50 MHz.

NTIA considered the federal agencies’ input toward the development of these U.S. proposals for WRC-19.  NTIA forwards this package for your consideration and review.  If you have any questions, please contact Mr. Charles Glass, our WRC Coordinator, who can be reached at (202) 482-1896 or cglass@ntia.doc.gov.

Sincerely,

Peter A. Tenhula

Acting Associate Administrator

Office of Spectrum Management

Enclosures (3)

UNITED STATES OF AMERICA

PROPOSALS FOR THE WORK OF THE CONFERENCE

Agenda Item 1.16:  To consider issues related to wireless access systems, including radio local area networks (WAS/RLAN), in the frequency bands between 5 150 MHz and 5 925 MHz, and take the appropriate regulatory actions, including additional spectrum allocations to the mobile service, in accordance with Resolution 239 (WRC-15).

Background Information:  Resolution 239 (WRC-15) calls for ITU-R to study WAS/RLAN technical characteristics and operational requirements in the 5 GHz frequency range.  The resolution also calls for ITU-R to perform sharing and compatibility studies between WAS/RLAN applications and incumbent services in these frequency bands:

- 5 150-5 350 MHz

- 5 350-5 470 MHz

- 5 725-5 850 MHz

- 5 850-5 925 MHz

The studies are to be performed to: 1) ensure the protection of incumbent services, including their current and planned use; 2) consider enabling outdoor WAS/RLAN operations in the band 5 150-5 350 MHz; 3) potentially add mobile service allocations to accommodate WAS/RLAN operations in the 5 350-5 470 MHz and 5 725-5 850 MHz bands; and 4) identify potential WAS/RLAN use in the 
5 850-5 925 MHz band.

This document proposes NOC for the 5 250-5 350 MHz, 5 350-5 470 MHz, 5 725-5 850 MHz, and  
5 850-5 925 MHz bands.

Proposal:

Article 5

Frequency Allocations

Section IV – Table of Frequency Allocations

(See No. 2.1)

NOC			USA/1.16/1

| 5 250-5 570 MHz |  |  |
| --- | --- | --- |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH 5.447D 5.447E 5.448 5.448A |  |  |
| 5 255-5 350 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH (active) 5.447E 5.448 5.448A |  |  |
| … |  |  |

Reasons:  Since the adoption of Resolution 229 (Rev. WRC-12) at WRC-03, millions of WAS/RLAN (such as Wi-Fi) devices have been deployed in the band 5 250-5 350 MHz.  Studies in response to invite c of Resolution 229 have shown that changing the WAS/RLAN operating conditions in the band 5 250-5 350 MHz would not ensure protection of incumbent radiodetermination services and EESS (active) sensors.  Furthermore, the studies confirmed that the current WAS/RLAN operating conditions in the band 5 250-5 350 MHz are sufficient for WAS/RLAN users.

NOC			USA/1.16/2

| 5 250-5 570 MHz |  |  |
| --- | --- | --- |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 5 350-5 460 EARTH EXPLORATION-SATELLITE (active) 5.448B RADIOLOCATION 5.448D AERONAUTICAL RADIONAVIGATION 5.449 SPACE RESEARCH (active) 5.448C |  |  |
| 5 460-5 470 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION 5.448D RADIONAVIGATION 5.449 SPACE RESEARCH (active) 5.448B |  |  |
| … |  |  |

Reasons:  Previous ITU-R studies show that sharing between WAS/RLAN and the EESS (active) systems or between WAS/RLAN and different radar systems in the band 5 350-5 470 MHz would not be feasible unless WAS/RLAN implement additional mitigation measures.  After further review of currently available mitigation measures, study results show that there are no feasible mitigation techniques to facilitate sharing between WAS/RLAN and EESS (active) or between WAS/RLAN and radar systems in the 5 350-5 470 MHz band.

NOC			USA/1.16/3

| 5 570-6 700 MHz |  |  |
| --- | --- | --- |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 5 725-5 830 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur | 5 725-5 830 RADIOLOCATION Amateur |  |
| 5.150 5.451 5.453 5.455 | 5.150 5.453 5.455 |  |
| 5 830-5 850 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) | 5 830-5 850 RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) |  |
| 5.150 5.451 5.453 5.455 | 5.150 5.453 5.455 |  |
| … |  |  |

Reasons:  WAS/RLAN already operate in various countries throughout the world within the frequency band 5 725-5 850 MHz.  In Region 2, the band 5 725-5 825 MHz is also used by WAS/RLAN.

NOC			USA/1.16/4

| 5 570-6 700 MHz |  |  |
| --- | --- | --- |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Amateur Radiolocation | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Radiolocation |
| 5.150 | 5.150 | 5.150 |
| … |  |  |

Reasons:  The mobile service is co-primary in the 5 850-5 925 MHz band.  Various countries have already implemented RLAN applications under the mobile allocation in this band.  Therefore, any sharing analysis carried out under this agenda item should not prejudice usages of the mobile service and should not consider imposing any additional constraints on other allocated services in the band.  Since multiple applications are being considered for implementation in this band, it is premature to consider regulatory changes at this time.

____________________

UNITED STATES OF AMERICA

PROPOSALS FOR THE WORK OF THE CONFERENCE

Agenda Item 9.1 issue 9.1.5:  Consideration of the technical and regulatory impacts of referencing Recommendations ITU-R M.1638-1 and ITU-R M.1849-1 in Nos. 5.447F and 5.450A of the Radio Regulations (WRC-15).

Background Information:  WRC-03 allocated the 5 150-5 350 MHz and 5 470-5 725 MHz frequency bands to the mobile service on a primary basis for the implementation of Wireless Access Systems (WAS) including Radio Local Area Networks (RLANs) subject to Resolution 229 (Rev. WRC-12).  WRC-03 also decided that the radiolocation service, the Earth exploration-satellite service (active), the space research service (active) (No. 5.447F), and the radiodetermination service (No. 5.450A) shall not impose on the mobile service more stringent protection criteria, based on system characteristics and interference criteria, than those stated in Recommendations ITU-R M.1638-0 and ITU-R RS.1632-0, which are incorporated in the Radio Regulations by reference.

Since WRC-03, millions of RLAN devices have been deployed worldwide.  ITU-R revised Recommendation M.1638-0 during the WRC-15 study cycle.  The revised recommendation (M.1638-1) included several new radars with different system characteristics, and removed the technical characteristics and protection criteria for ground-based meteorological radars, which were placed in Recommendation ITU-R M.1849-1 instead.  Several new meteorological radars were also added to Recommendation ITU-R M.1849-1 during this revision process.

This document proposes to modify Nos. 5.447F and 5.450A to remove the references to Recommendations ITU-R M.1638-1 and M.1849-1 while retaining the same regulatory status for the allocated services.

Proposal:

Article 5

Frequency Allocations

Section IV – Table of Frequency Allocations

(See No. 2.1)

MOD			USA/9.1.5/1

5.447F	In the frequency band 5 250-5 350 MHz, stations in the mobile service shall not claim protection from the radiolocation service, the Earth exploration-satellite service (active) and the space research service (active) while the radiolocation service, the Earth exploration–satellite service (active) and the space research service (active) shall not impose more stringent technical and operational limits upon the mobile service than those in 5.446A.

Reasons:  The proposed changes will allow the allocated services to retain the same regulatory status without referencing the subject recommendations in the footnotes of the Radio Regulations.  As the responsible ITU-R group routinely updates radar characteristics based on administration proposals, this solution will ensure timely consideration of radar characteristics changes without requiring conference action to modify the footnotes that reference the recommendations.

MOD			USA/9.1.5/2

5.450A	In the frequency band 5 470-5 725 MHz, stations in the mobile service shall not claim protection from radiodetermination service while the radiodetermination service shall not impose more stringent technical and operational limits upon the mobile service than those in 5.446A.

Reasons:  The proposed changes will allow the allocated services to retain the same regulatory status without referencing the subject recommendations in the footnotes of the Radio Regulations.  As the responsible ITU-R group routinely updates radar characteristics based on administration proposals, this solution will ensure timely consideration of radar characteristics changes without requiring conference action to modify the footnotes that reference the recommendations.

____________________

UNITED STATES OF AMERICA

PROPOSALS FOR THE WORK OF THE CONFERENCE

Agenda Item 10:	to recommend to the Council items for inclusion in the agenda for the next WRC, and to give its views on the preliminary agenda for the subsequent conference and on possible agenda items for future conferences, in accordance with Article 7 of the Convention.

Background Information:  At WRC-15 CITEL put forward Inter-American Proposals for an item to be placed on the draft agenda of WRC-23 and a resolution calling for supporting studies.  These proposals, agreed at WRC-15, supported an allocation to the Earth exploration-satellite (active) service in the frequency range 40-50 MHz.  The draft agenda item is number 2.2 in Resolution 810 (WRC-15), and the supporting studies are called for in Resolution 656 (WRC-15).  The United States continues to support the retention of this item on the agenda of WRC-23.

There is an interest among space agencies in using active spaceborne sensors in the 40-50 MHz frequency range for measurements of the Earth’s subsurface to provide radar maps of subsurface scattering layers with the intent to locate water/ice deposits.  Measurements at the 40-50 MHz frequency range allow the discernment of details at more than 30 meters below the surface of the Earth for favorable ground conditions. Use of frequencies below 40-50 MHz would require larger antenna, which would present difficulties to spaceborne missions implementing this application.  Use of frequencies above 40-50 MHz would reduce the depth at which the spaceborne radar sounder could provide measurements.  Use of a frequency range other than 40-50 MHz would require new aeronautical campaigns at the different frequency in order to assess and calibrate the measurements at that frequency for use in a spaceborne radar sounder mission.

The information obtained from a spaceborne radar sounder operating in the 40-50 MHz frequency range would be of great value to ongoing global climate change studies, and to administrations in their assessment of below surface water resources within their territories.  Repetitive measurements of worldwide subsurface water deposits can only be practically implemented using spaceborne active sensors.

The 40-50 MHz frequency range is allocated to the fixed, mobile and broadcasting services on a primary basis.  The uses of the 40.98 to 41.015 MHz frequency band by space research service are on secondary basis. Country footnotes in the Table of Frequency Allocations for the 40-50 MHz frequency range provide primary allocations for aeronautical navigation and radiolocation services in certain parts of the world.  Recommendation ITU-R RS.2042-0 provides typical technical and operating characteristics for spaceborne radar sounder systems in the 40-50 MHz band for use in interference and compatibility studies.

This future conference agenda item proposes to study the compatibility of spaceborne radar sounder operations in the 40-50 MHz frequency range with the existing allocated services.  In addition, it would investigate a potential modification to the Table of Frequency Allocations to add an allocation to the Earth exploration-satellite service (active).  This allocation would allow for the operation of spaceborne radar sounder systems in the 40-50 MHz frequency range.

Proposals:

ADD	USA/10/1

Draft New Resolution [USA-AXXX]

Agenda for the 2023 World Radiocommunication Conference

The World Radiocommunication Conference (Sharm el-Sheikh, 2019),

...

X.X1	to conduct, and complete in time for WRC-23, studies for a possible new allocation to the Earth exploration-satellite (active) service for spaceborne radar sounders within the range of frequencies around 45 MHz, taking into account the protection of incumbent services, in accordance with Resolution 656 (WRC-15);

Reasons:	To conduct studies to examine the compatibility of spaceborne radar sounder operations in the 40-50 MHz frequency range with existing allocated services and to potentially modify the Table of Frequency Allocations to add an allocation to the Earth exploration-satellite service (active) allowing for the operation of spaceborne radar sounder systems in the 40-50 MHz frequency range.

NOC 	USA/10/2

RESOLUTION 656 (WRC-15)

Possible allocation to the Earth exploration-satellite service (active) for spaceborne radar sounders in the range of frequencies around 45 MHz

The World Radiocommunication Conference (Geneva, 2015),

Reasons:	Resolution 656 (WRC-15) calls for the studies supporting this agenda item.  Consequential changes are unnecessary.

SUP	USA/10/3

RESOLUTION 810 (WRC-15)

Preliminary agenda for the 2023 World Radiocommunication Conference

The World Radiocommunication Conference (Geneva, 2015),

Reasons:	This Resolution must be suppressed, as WRC-19 will create a new Resolution that will include the agenda for WRC-23.