UNITED STATES OF AMERICA

DRAFT PROPOSAL FOR THE WORK OF THE CONFERENCE

Agenda Item 1.16:  to consider issues related to wireless access systems, including radio local area networks (WAS/RLAN), in the frequency bands between 5 150 MHz and 5 925 MHz, and take the appropriate regulatory actions, including additional spectrum allocations to the mobile service, in accordance with Resolution 239 (WRC-15)

Background Information:

In the early-to-mid 1990s, the ITU authorized non-geostationary satellite systems in the Mobile-Satellite Service (MSS) under revisions to the ITU-R Radio Regulations. Specifically, in 1992 the ITU adopted allocations for user links in the L and S bands, and in 1995 it adopted allocations for the feeder links between gateway earth stations and the NGSO satellites. The international feeder uplink allocations were made in the 5.1 and 29.2 GHz ranges, while the feeder downlink allocations were made in the 7 and 19 .5 GHz ranges. MSS operators have been licensed by different Administrations to use the 5150-5250 MHz range for feeder uplinks from earth stations to MSS satellites. These feeder uplinks at 5150-5250 MHz have been in continuous use globally since 1998. Two MSS operator currently use the 5150-5250 MHz band for this purpose. These MSS systems provide vital communication links to underserved areas and are often the only communication link available in these remote areas.

In Report and Order FCC 97-5 in 1997, the FCC authorized unlicensed Radio Local Area Networks (RLANs) known as “Unlicensed National Information Infrastructure” (U-NII). In this Report and Order, the FCC established the “U-NII-1” band at 5150-5250 MHz. In order to protect other radio services at 5150-5250 MHz, including fixed-satellite service (FSS) transmissions used to provide feeder uplinks for non-geostationary MSS systems, the FCC adopted a maximum equivalent isotropic radiated power (EIRP) of 200 milliwatts (mW) and restricted RLAN operations in the U-NII-1 band to indoor use.

Following a year-long rulemaking proceeding, the FCC in 2014 adopted revised rules in the U-NII-1 band that permit outdoor RLAN operations at increased power levels. The rules adopted in FCC Report and Order FCC 14-30 generally permit indoor and outdoor RLAN operations in the 5150 – 5250 MHz band at up to 1 Watt conducted or 4 Watt E.I.R.P., except that operations with antenna elevation angles in excess of 30 degrees from the horizon must not exceed 125 mW E.I.R.P. These rules were intended to prevent harmful interference to MSS Earth-to-space communications by limiting the aggregate noise received by the satellite. U.S. RLAN interests and the MSS operator participating in the FCC’s regulatory proceeding agreed to this approach to sharing, with the MSS operator’s agreement conditioned on the existence of a remedial mechanism to limit any interference to MSS to an acceptable level. In this Report and Order, the FCC stated that licensed MSS operations are protected from harmful interference from unlicensed terrestrial wireless operations.  The U.S. Administration also recognized the ability of the participating MSS operator to measure interference in the feeder uplink band, and stated that remedial action would be taken to reduce or eliminate any harmful interference to licensed MSS operations.

The MSS operator’s measurement of the noise level over the USA in the 5091-5250 MHz band began in 2014 and is currently ongoing. This MSS operator’s measurements have shown a significant increase in interference to its feeder uplinks and MSS system since early 2017. This interference to the MSS operator’s satellite service has resulted in reduced MSS capacity, reduction of MSS coverage area, and an increased burden on the spacecraft bus power of the MSS satellites. These negative effects are expected to become greater over time as outdoor RLAN operations expand and the noise level in the 5091-5250 MHz band over the U.S. continues to rise. Although the MSS operator’s measurements cover spectrum outside the 5150-5250 MHz frequency range (at 5091-5150 MHz), exhaustive research has not revealed any material interference sources at 5091-5150 MHz. In addition, the MSS operator has conducted similar measurements of the noise level at 5091-5250 MHz over Europe, Central and South America, and Australia, and these measurements have shown no similar increase in interference. The MSS operator has provided the results of its interference measurements to the FCC.

Since the FCC’s adoption of these more permissive regulations for outdoor RLAN operations at 5150–5250 MHz, other countries have authorized similar outdoor RLAN deployments, but only with regulatory constraints such as temporary, provisional licenses and registration of RLAN transmitter locations.

Studies are currently ongoing in the ITU-R to determine whether it is feasible to permit co-channel operation of outdoor RLAN transmitters and MSS feeder uplink operations at 5150-5250 MHz without causing unacceptable interference to MSS satellite receivers. The MSS operator conducting interference measurements has presented its measurement results to the ITU-R. Further, it has petitioned the FCC to open an inquiry seeking comment on the feasibility of continued sharing of the 5150-5250 MHz band between the MSS and outdoor RLAN transmitters. Prior to the 2014 Rule change, in the USA, that allowed outdoor deployment and increased transmitter power, RLANs and MSS feeder links successfully shared the 5150-5250 MHz band for nearly 16 years.

In light of these facts, incorporation of the revisions to the Radio Regulations that permit outdoor deployment and increased transmitter power is not consistent with the protection of incumbent services from unacceptable interference.

NOC	USA/1.16/1

RESOLUTION 229 (Rev.WRC-19)

Use of the bands 5 150-5 250 MHz, 5 250-5 350 MHz and 5 470-5 725 MHz 
by the mobile service for the implementation of wireless access systems 
including radio local area networks

NOC	USA/1.16/2

5.446A	The use of the bands 5 150-5 350 MHz and 5 470-5 725 MHz by the stations in the mobile, except aeronautical mobile, service shall be in accordance with Resolution 229 (Rev.WRC-1215)

Amendment of the Commission’s Rules to Provide for Operation of Unlicensed NII Devices in the 5 GHz Frequency Range, Report and Order, 12 FCC Rcd 1576, FCC 97-5 (1997) (“1997 U-NII Order”).

Revision of Part 15 of the Commission’s Rules to Permit Unlicensed National Information Infrastructure (U-NII) Devices in the 5 GHz Band, First Report and Order, FCC 14-30, 29 FCC Rcd. 4127 (2014).

47 C.F.R. § 15.407(a)(1)(i).

MSS interests feared that such a drastic change in operating conditions - outdoor deployment and an 80 times increase in power level - would result in disruptive interference to feeder uplink operations. This EIRP coupled with the loss due to indoor deployment of the U-NII devices, standardized at 50 times, would result in an overall EIRP increase of 4000 times or 36 dB.