September 6, 2018

Mr. Tom Sullivan

Chief, International Bureau

Federal Communications Commission

445 12th Street, SW

Washington, DC  20554

Dear Mr. Sullivan:

The National Telecommunications and Information Administration (NTIA), on behalf of the Executive Branch agencies, approves the release of the enclosed proposals for the 2019 World Radiocommunication Conference (WRC-19) which address:

- Agenda item 1.13 – International Mobile Telecommunications (IMT) studies between 24.25 GHz and 86 GHz with proposals for the 31.8-33.4 GHz, 45.5-47 GHz, and 47-47.2 GHz bands

- Agenda item 9.1, issue 9.1.6 – Urgent studies for Wireless Power Transfer (WPT) for electric vehicles

NTIA considered the federal agencies’ input toward the development of these U.S. proposals for WRC-19. NTIA forwards this package for your consideration and review. If you have any questions, please contact Mr. Charles Glass, our WRC Coordinator, who can be reached at (202) 482-1896 or cglass@ntia.doc.gov.

Sincerely,

Peter A. Tenhula

Acting Associate Administrator

Office of Spectrum Management

Enclosures (2)

Attachment 1

United States

PROPOSALS FOR THE WORK OF THE CONFERENCE

Agenda item 1.13

1.13	to consider identification of frequency bands for the future development of International Mobile Telecommunications (IMT), including possible additional allocations to the mobile service on a primary basis, in accordance with Resolution 238 (WRC-15);

Background Information: Resolution 238 (WRC-15) invites ITU-R to conduct and complete in time for WRC-19 appropriate studies to determine the spectrum needs for the terrestrial component of IMT in the frequency range between 24.25 GHz and 86 GHz, as well as sharing and compatibility studies, taking into account the protection of services to which the frequency band is allocated on a primary basis, for the frequency bands:

–	24.25-27.5 GHz, 37-40.5 GHz, 42.5-43.5 GHz, 45.5-47 GHz, 47.2-50.2 GHz, 50.4-52.6 GHz, 66-76 GHz and 81-86 GHz, which have allocations to the mobile service on a primary basis; and

–	31.8-33.4 GHz, 40.5-42.5 GHz and 47-47.2 GHz, which may require additional allocations to the mobile service on a primary basis.

The Resolution further invites WRC-19 to consider, based on the results of these studies, additional spectrum allocations to the mobile service on a primary basis and identification of frequency bands for the terrestrial component of IMT. The bands to be considered are limited to part or all of the bands listed above.

31.8-33.4 GHz: Sharing and compatibility studies were conducted between IMT-2020 systems and the following systems in the 31.8-33.4 GHz range: radionavigation service (RNS), space research service (Deep Space) (SRS), Earth exploration-satellite (EESS) (passive), and radio astronomy service (RAS). The results are briefly summarized as follows:

- RNS: The studies show that sharing between IMT-2020 systems and the radionavigation service within 31.8-33.4 GHz is not feasible

- SRS: The study results indicate that the separation distances needed to protect these particular facilities are relatively small; consequently, the protection of these stations could be considered on a national or bilateral/multilateral level.

- EESS (passive):  The studies show interference exceedance.

- RAS: The studies show that separation distances of 19-49 km are needed.

45.5-47 GHz: No studies were performed between IMT-2020 systems in this band and the relevant incumbent services.

47-47.2 GHz: No studies were performed between IMT-2020 systems in this band and the relevant incumbent services.

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

NOC	USA/4773A13/1

29.9-34.2 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 29.9-30 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 5.543 5.525 5.526 5.527 5.538 5.540 5.542 |  |  |
| 30-31 FIXED-SATELLITE (Earth-to-space) 5.338A MOBILE-SATELLITE (Earth-to-space) Standard frequency and time signal-satellite (space-to-Earth) 5.542 |  |  |
| 31-31.3 FIXED 5.338A 5.543A MOBILE Standard frequency and time signal-satellite (space-to-Earth) Space research 5.544 5.545 5.149 |  |  |
| 31.3-31.5 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |
| 31.5-31.8 EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile | 31.5-31.8 EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | 31.5-31.8 EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile |
| 5.149 5.546 | 5.340 | 5.149 |
| 31.8-32 FIXED 5.547A RADIONAVIGATION SPACE RESEARCH (deep space) (space-to-Earth) 5.547 5.547B 5.548 |  |  |
| 32-32.3 FIXED 5.547A RADIONAVIGATION SPACE RESEARCH (deep space) (space-to-Earth) 5.547 5.547C 5.548 |  |  |
| 32.3-33 FIXED 5.547A INTER-SATELLITE RADIONAVIGATION 5.547 5.547D 5.548 |  |  |
| 33-33.4 FIXED 5.547A RADIONAVIGATION 5.547 5.547E |  |  |
| 33.4-34.2 RADIOLOCATION 5.549 |  |  |

Reasons:	The studies show that sharing is not feasible between IMT-2020 systems in 31.8-33.4 GHz and at least one of the incumbent services.

NOC	USA/4773A13/2

40-47.5 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 40-40.5 EARTH EXPLORATION-SATELLITE (Earth-to-space) FIXED FIXED-SATELLITE (space-to-Earth) 5.516B MOBILE MOBILE-SATELLITE (space-to-Earth) SPACE RESEARCH (Earth-to-space) Earth exploration-satellite (space-to-Earth) |  |  |
| 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) 5.516B BROADCASTING BROADCASTING-SATELLITE Mobile Mobile-satellite (space-to-Earth) 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 |
| 41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) 5.516B BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F 5.551H 5.551I |  |  |
| 42.5-43.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 MOBILE except aeronautical mobile RADIO ASTRONOMY 5.149 5.547 |  |  |
| 43.5-47 MOBILE 5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.554 |  |  |
| 47-47.2 AMATEUR AMATEUR-SATELLITE |  |  |
| 47.2-47.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 MOBILE 5.552A |  |  |

Reasons:	No studies were performed between IMT-2020 systems in the 45.5-47 GHz band or the 47-47.2 GHz band and the incumbent services; therefore, it has not been demonstrated that the incumbent services can be protected, as required by Resolution 238 (WRC-15).

SUP	USA/4773A13/3

RESOLUTION 238 (WRC-15)

Studies on frequency-related matters for International Mobile Telecommunications identification including possible additional 
allocations to the mobile services on a primary basis in portion(s) 
of the frequency range between 24.25 and 86 GHz for the future 
development of International Mobile Telecommunications 
for 2020 and beyond

The World Radiocommunication Conference (Geneva, 2015),

Reasons:	The studies called for under the agenda item for most of the bands have been completed. The fact that no studies on the 45.5-47 GHz band or the 47-47.2 GHz band, which were called for under the agenda item, have been performed indicates no interest in these bands for IMT-2020. There is no need to retain Resolution 238 (WRC-15).

____________________________________________________

Attachment 2

United States

PROPOSALS FOR THE WORK OF THE CONFERENCE

Agenda item 9, Issue 9.1.6

9	to consider and approve the Report of the Director of the Radiocommunication Bureau, in accordance with Article 7 of the Convention:

9.1	on the activities of the Radiocommunication Sector since WRC-15;

9.1 (9.1.6) 	Resolution 958 (WRC-15) - Annex item 1) Studies concerning Wireless Power Transmission (WPT) for electric vehicles: a) to assess the impact of WPT for electric vehicles on radiocommunication services;  b) to study suitable harmonized frequency ranges which would minimize the impact on radiocommunication services from WPT for electrical vehicles. These studies should take into account that the International Electrotechnical Commission (IEC), the International Organization for Standardization (ISO) and the Society of Automotive Engineers (SAE) are in the process of approving standards intended for global and regional harmonization of WPT technologies for electric vehicles;

Background Information:

WPT technologies are being developed worldwide to support many applications to transfer power without employing any wired connection. WPT for electric vehicles (WPT EV) applications are part of the WPT development efforts. The objectives of WPT EV developments are to reduce EV battery size, increase the driving distance between charging, and improve the practicality by making EV charging stations more available and accessible for consumers, governments, and other public transit authorities.

WRC-15 adopted Resolution 958 (WRC-15) to examine the possible impact of WPT EV charging on the radiocommunication services. Annex item 1 of the resolution requests, as a matter of urgency, that ITU-R study the impact of WPT EV on radiocommunication services and study suitable harmonized frequency ranges that would minimize this impact. ITU-R identified two frequency ranges for EV charging that might be suitable for harmonization: the 79-90 kHz frequency range for medium power and 19-25 kHz for high power. ITU-R conducted a number of impact studies between WPT EV and radiocommunication services to assess the potential interference impact of WPT EV applications operating within the 19-25 kHz and 79-90 kHz frequency ranges. These analyses are included in Report ITU-R SM.2303, [the working document towards a preliminary draft new] Report SM.[WPT_SPEC_MNGM], and [the working document towards a preliminary draft revision of] Recommendation ITU-R SM.2110. Upon approval of these documents, the urgent studies requested in Resolution 958 (WRC-15) Annex 1 (b) will have been addressed.

The power level of WPT EV selected to charge an EV battery will depend on the circumstances and requirements of the users. In most cases for electric vehicles, the charger will be located in a residential setting, with power levels between 3.3 kW and 22 kW. It will likely take a number of hours to fully charge the EV battery within this power range. These characteristics provide a suitable balance between domestic electrical load limitations, charging time considerations, and other regulations governing electrical installations for domestic use. However, where users need higher charging rates to satisfy specific heavy-duty vehicle use (such as buses, trams, and trucks), much higher power levels may be needed to reduce the charging times to satisfy commercial and operational demands. A minimum of 75 kW equivalent charging power is likely to be required initially, and the requirement will increase to several hundred kW for certain intensively used heavy-duty vehicles.

WPT does not have any status in the Radio Regulations; therefore, it should not cause interference to radiocommunication services. For the frequency ranges proposed for harmonized WPT EV use, Nos. 15.12 and 15.12.1 apply to ensure that electrical apparatus of any kind does not cause harmful interference to a radiocommunication service and that administrations should be guided by the latest ITU-R Recommendations in this matter. Recommendation ITU-R SM.2110 provides further information on the regulatory status of WPT.

NOC	USA/4775A21A6/1

ARTICLE 5

Frequency allocations

Reasons:	Based on ITU-R studies conducted and analyses made per Resolution 958 (WRC-15) Annex 1, the impact on radiocommunication services from WPT EV can be minimized by the application of power limits, separation distances, and careful control of harmonic energy, including the selection of WPT EV fundamental frequency within the frequency ranges.  Further, [working document towards a preliminary draft new] Report ITU-R SM.[WPT_SPEC_MNGM] provides the required power limitations and associated frequency ranges.

Therefore, it is unnecessary to make any modifications to the Radio Regulations or to the regulatory status of WPT for the introduction of WPT EV applications. These applications can be developed, deployed, and used effectively within the existing regulatory framework in accordance with Nos. 15.12 and 15.12.1 and regional and individual administration’s authorization requirements. Additional guidance for administrations wishing to deploy WPT EV applications can be found in ITU-R Reports and Recommendations, including [the working document towards a preliminary draft new] Report ITU-R SM.[WPT_SPEC_MNGM] and the latest version of Recommendation ITU-R SM. 2110.

RESOLUTION 958 (WRC-15)

Urgent studies required in preparation for the
2019 World Radiocommunication Conference

SUP	USA/4775A21A6/2

ANNEX TO RESOLUTION 958 (WRC-15)

Urgent studies required in preparation for the 
2019 World Radiocommunication Conference

1)	Studies concerning Wireless Power Transmission (WPT) for electric vehicles:

a)	to assess the impact of WPT for electric vehicles on radiocommunication services;

b)	to study suitable harmonized frequency ranges which would minimize the impact on radiocommunication services from WPT for electrical vehicles.

These studies should take into account that the International Electrotechnical Commission (IEC), the International Organization for Standardization (ISO) and the Society of Automotive Engineers (SAE) are in the process of approving standards intended for global and regional harmonization of WPT technologies for electric vehicles.

2)	Studies to examine:

a)	whether there is a need for possible additional measures in order to limit uplink transmissions of terminals to those authorized terminals in accordance with No. 18.1;

b)	the possible methods that will assist administrations in managing the unauthorized operation of earth station terminals deployed within its territory, as a tool to guide their national spectrum management programme, in accordance with Resolution ITU-R 64 (RA-15).

3)	Studies on the technical and operational aspects of radio networks and systems, as well as spectrum needed, including possible harmonized use of spectrum to support the implementation of narrowband and broadband machine-type communication infrastructures, in order to develop Recommendations, Reports and/or Handbooks, as appropriate, and to take appropriate actions within the ITU Radiocommunication Sector (ITU-R) scope of work

Reasons:	The studies called for under the Resolution have been completed. No changes to the Radio Regulations are required. Further studies can be carried out under normal ITU-R procedures in the relevant working parties.

____________________________________________________

“When conducting studies in the frequency band 24.5-27.5 GHz, to take into account the need to ensure the protection of existing earth stations and the deployment of future receiving earth stations under the EESS (space-to-Earth) and SRS (space-to-Earth) allocation in the frequency band 25.5-27 GHz.”

The fundamental frequency selection may need to take into account regional differences.